| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: A7ME02

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

I B.Tech I Semester Regular Examinations December-2025

COMPUTER AIDED ENGINEERING DRAWING

(Common to ECE & EEE)

Time: 3 Hours.								Max. Marks: 60

Note: Answer any one question from each unit. Each question carries 12 marks and may have a, b, c as

sub questions.

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Discuss the basic drawing commands (line, circle, arc, polygon, polyline) and modification commands (trim, extend, fillet, chamfer). | 6 | 1 | 6 |
|  | b) | Explain the process of isometric drawing creation in CAD. Discuss isometric grids, snap settings, and isoplane selection. | 6 | 1 | 2 |
| OR |  |  |  |  |  |
| 2 |  | Using AutoCAD, draw a rectangle (100 mm × 60 mm), and inside it construct a circle of diameter 30 mm centered. List all the draw commands used and steps followed. | 12 | 1 | 3 |
| 3 | a) | Draw Hyperbola when the distance of its focus from its directrix is 50 mm and eccentricity is 3/2. | 6 | 2 | 1 |
|  | b) | Draw a Hypocycloid of a Circle of 40 mm Diameter which rolls inside the another circle of 200 mm Diameter for one Revolution | 6 | 2 | 3 |
| OR |  |  |  |  |  |
| 4 |  | Draw an Ellipse Using Auto CAD when the distance of its focus from its directrix is 50 mm and eccentricity is 2/3. Also, draw a tangent and a normal to the ellipse at a Point 70 mm away from the directrix. | 12 | 2 | 3 |
| 5 |  | Draw the projections of the following points on the same ground line, keeping the projectors 25 mm apart by Using Auto CAD - A is in the H.P. and 20 mm behind the V.P. - B is 40 mm above the H.P. and 25 mm in front of the V.P. - C is in the V.P. and 40 mm above the H.P. - D is 25 mm below the H.P. and 25 mm behind the V.P. - E is 15 mm above the H.P. and 50 mm behind the V.P. - F is 40 mm below the H.P. and 25 mm in front of the V.P. vii.G is in both the H.P. and the V.P. | 12 | 3 | 3 |
| OR |  |  |  |  |  |
| 6 | a) | A 70 mm long line PQ, has its end P 20 mm above the H.P. and 30 mm in front of the V.P. The line is inclined at 45° to the H.P. and 30° to the V.P. Draw its projections by Using Auto CAD | 6 | 3 | 4 |
|  | Page 1 of 2 |  |  |  |  |
|  | b) | A Square Plane of side 40 mm has its surface parallel to and 20 mm above the H.P. By Using Auto CAD Draw its projections when - a side is parallel to the V.P., - One side is inclined at 30°to the V.P. - All sides are equally inclined to the V.P. | 6 | 3 | 4 |
| 7 |  | A square prism of base side 40 mm and axis 60 mm is resting on its base on the ground By Using Auto CAD, Draw its projections when (i) a face is perpendicular to the V.P., (ii) a face is inclined at 30° to the V.P., (iii) all the faces are equally inclined to the V.P | 12 | 4 | 4 |
| OR |  |  |  |  |  |
| 8 | a) | A Square pyramid of base side 40 mm and axis 55 mm is resting on one of its triangular faces on the H.P. A vertical plane containing the axis is inclined at 45° to the V.P. Draw its projections by Using Auto CAD | 6 | 4 | 4 |
|  | b) | A square prism of base side 30 mm and axis 60 mm is resting on its base on the H.P. with a rectangular face parallel to the V.P. Draw the development of the prism by Using Auto CAD. | 6 | 4 | 4 |
| 9 |  | Convert the isometric drawings into orthographic view (Front, side & Top views) with proper dimensions. | 12 | 5 | 1 |
| OR |  |  |  |  |  |
| 10 |  | Translate given orthographic view into isometric view | 12 | 5 | 5 |

---oo0oo--- 					Page 2 of 2