| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	A7ME02

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

I B.Tech I Semester Regular Examinations December-2025

COMPUTER AIDED ENGINEERING DRAWING

(CSD)

Time: 3 Hours.								              Max. Marks: 60

Note: Answer any one question from each unit. Each question carries 12 marks and may have a, b, c as

sub questions.

| 1 |  |  |  | M |  | CO | BL |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Draw the following fig. by using Auto CAD commands. |  | 12 |  | 1 | 1 |
| OR |  |  |  |  |  |  |  |
| 2 |  | Draw the following fig. by using Auto CAD commands. |  | 12 |  | 1 | 1 |
| 3 | Draw Locus of a Point on the periphery of a circle which rolls on straight line path. Take circle diameter as 50 mm and draw tangent and normal at a distance of 35 mm from the base. |  | 12 |  | 2 |  | 1 |
| Page 1 of 2 |  |  |  |  |  |  |  |
| OR |  |  |  |  |  |  |  |
| 4 |  | Draw an Ellipse when the distance of its focus from its directrix is 50 mm and eccentricity is 2/3. Also, draw a tangent and a normal to the ellipse at a Point 70 mm away from the directrix. |  | 12 |  | 2 | 1 |
| 5 |  | A pentagonal plane with a 25 mm side rests on the H.P. on one of its corners with its surface perpendicular to the V.P. and inclined at 300 to the H.P. Draw its projections when the side opposite to the corner makes 450 to the V.P |  | 12 |  | 3 | 3 |
| OR |  |  |  |  |  |  |  |
| 6 |  | Draw the projections of a straight line AB 80 mm long inclined at 450to the HP and 300 to the VP with the end A in the HP and VP. |  | 12 |  | 3 | 4 |
| 7 |  | A hexagonal pyramid of base side 30mm and axis 60 mm has an edge of its base on the ground. Its axis is inclined at 300 to the ground and parallel to the VP. Draw its projections. |  | 12 |  | 4 | 4 |
| OR |  |  |  |  |  |  |  |
| 8 |  | A square prism of base side 30mm and axis 60mm is resting on its base on the H.P. with a rectangular face parallel to the V.P. the cutting plan pass through midpoint of the axis with 450 inclinations. Draw the development of the prism. |  | 12 |  | 4 | 3 |
| 9 |  | Draw the isometric view of a pentagonal prism of base side 30 and axis 55 mm. The prism is resting on its base on the H.P. with an edge of the base parallel to the V.P. |  | 12 |  | 5 | 4 |
| OR |  |  |  |  |  |  |  |
| 10 |  | Draw the orthographic Projection of the following object ( Front view, Top View & Left Hand side View) |  | 12 |  | 5 | 1 |

---oo0oo---

Page 2 of 2