| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: A7ME02

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

I B.Tech I Semester Regular Examinations December-2025/January-2026

COMPUTER AIDED ENGINEERING DRAWING

(CSM)

Time: 3 Hours.								Max. Marks: 60

Note:  Answer any one question from each unit. Each question 
           carries 12 marks and may have a, b, c as sub questions.

| 1 |  |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- | --- |
|  | a) |  | Using CAD, draw a rectangle 100 × 60 and apply FILLET R10 at all corners; show the command steps. | 6 | 1 | 3 |
|  | b) |  | In CAD, draw a bearing plate: rectangle 120 × 60, with two Ø20 holes whose centers are 30 mm from each short end and 30 mm from the long edge (centerline). Dimension fully. | 6 | 1 | 3 |
| OR |  |  |  |  |  |  |
| 2 |  |  | Draw the following diagram by using AutoCAD commands and dimension it. | 12 | 1 | 3 |
| 3 | a) | Construct a hyperbola by General Method for e = 3/2. Focus is 45 mm from directrix. |  | 6 | 2 | 5 |
|  | b) | Construct a hypocycloid for a rolling circle of radius 20 mm rolling inside a fixed circle of radius 60 mm (one complete cycle). |  | 6 | 2 | 5 |
| OR |  |  |  |  |  |  |
| 4 | a) |  | Construct an ellipse by General Method for e = 1/2. Focus is 40 mm from directrix. | 6 | 2 | 5 |
|  | b) |  | Construct an epicycloid for a rolling circle of radius 20 mm rolling on a fixed circle of radius 60 mm (one complete cycle) | 6 | 2 | 5 |
| 5 | a) |  | A point A is 30 mm above HP and 20 mm behind VP. Draw its projections and state its quadrant | 6 | 3 | 4 |
|  | b) |  | A regular hexagonal lamina (side 25 mm) is inclined 40° to HP and perpendicular to VP. Draw its projections. | 6 | 3 | 3 |
| OR Page 1 of 2 |  |  |  |  |  |  |
| 6 | a) |  | A line AB = 80 mm is parallel to VP and inclined 35° to HP. End A is 20 mm above HP and 25 mm in front of VP. Draw FV & TV and find apparent length in TV. | 6 | 3 | 3 |
|  | b) |  | A point B is 15 mm below HP and 45 mm in front of VP. Draw its projections and state its quadrant. | 6 | 3 | 4 |
| 7 | a) |  | A triangular prism (base side 35 mm, axis 70 mm) rests on HP on its base. Axis inclined 30° to HP and 45° to VP. Draw its projections. | 6 | 4 | 3 |
|  | b) |  | A square pyramid (base side 30 mm, axis 60 mm) rests on HP on its base. Axis inclined 30° to HP and 45° to VP. Draw its projections. | 6 | 4 | 3 |
| OR |  |  |  |  |  |  |
| 8 | a) |  | Develop the lateral surface with base of a cone: base Ø 50 mm, height 65 mm. | 6 | 4 | 4 |
|  | b) |  | Develop the lateral surface with base of a hexagonal prism: side 25 mm, height 60 mm | 6 | 4 | 4 |
| 9 | a) |  | Draw isometric view of a rectangular block 60 × 40 × 30 mm with a top central rectangular slot 30 × 20 × 10 mm deep. | 6 | 5 | 3 |
|  | b) |  | Convert isometric to orthographic: A cylinder Ø40 × 60 placed on a square plate 60 × 60 × 10 (cylinder centered). Draw FV, TV, SV. | 6 | 5 | 4 |
| OR |  |  |  |  |  |  |
| 10 |  |  | Draw the isometric view of given orthographic Projection. | 12 | 5 | 3 |

---oo0oo---

Page 2 of 2