SET-2

| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

I B.Tech I Semester Regular Examinations December-2025

SUBJECT NAME: ELECTRONIC DEVICES AND APPLICATIONS

(BRANCH)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			5 x 2M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Define static and dynamic resistance of a diode. | 2 | CO1 | L1 |
|  | b) | Find the efficiency of the half-wave rectifier | 2 | CO2 | L1 |
|  | c) | What is the thermal resistance? | 2 | CO3 | L1 |
|  | d) | Define the amplification factor in JFET. | 2 | CO4 | L1 |
|  | e) | Define the quantum efficiency of a photodiode. | 2 | CO5 | L1 |

PART- B 			5 x 10M = 50Marks

| 2 | UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | M | CO | BL |
|  | a) | Derive the expression of space charge capacitance CT for a step-graded junction. | 5 | CO1 | L2 |
|  | b) | Determine the diffusion capacitance due to holes in a Ge diode when the forward-biased current I = 26 mA. Given that the mean lifetime of a hole, τ = 20 µ sec at T= 3000 K. | 5 | CO1 | L3 |
| OR |  |  |  |  |  |
| 3 | a) | Explain the following terms. (i) Static Resistance (ii) Dynamic Resistance of a Diode with suitable figures | 5 | CO1 | L2 |
|  | b) | Calculate the contact potential V0 at room temperature. The resistivity of the P-region and N-region of a germanium diode are 6 Ω-cm and 4 Ω-cm, respectively. Given that q=1.602×10–19C, ni = 2.5 X 1013/cm3, µp=1800 cm2/V-s µn= 3800 cm2/V-s | 5 | CO1 | L3 |
| UNIT-II |  |  |  |  |  |
| 4 | a) | Determine ripple factor, rectification efficiency and TUF for the Bridge rectifier. | 5 | CO2 | L2 |
|  | b) | A full-wave rectifier with a centre-tapped transformer supplies a dc current of 100mA to a load resistance of R = 20. The secondary resistance of transformer is 1. Each diode has a forward resistance of 0.5. Determine the following. - rms value of the signal voltage across each half of the secondary. - dc power supplied to the load. - PIV rating for each diode. iV. ac power input to the rectifier | 5 | CO2 | L3 |
| OR |  |  |  |  |  |
| 5 | a) | Draw the circuit diagram of a full-wave rectifier and find out the Ripple Factor, Efficiency and Peak Factor | 5 | CO2 | L2 |
|  | b) | Determine the peak output voltage for a negative shunt clipper circuit as shown in the following figure. The input signal is sinusoidal of peak value 12V, the barrier voltage for the silicon diode is 0.7V, the series resistance is 500Ω, and the load resistance is 2.5k Ω. | 5 | CO2 | L3 |
| UNIT-III |  |  |  |  |  |
| 6 |  | Explain the operation of CE Configuration with the help of input & output characteristics and indicate the 3 operating regions. | 5 | CO3 | L2 |
|  |  | Solve the base, collector, and emitter currents and VCE for the CE circuit shown below. For VCC = 10V, VBB = 4V, RB = 200KΩ, RC = 2KΩ, VBE(on) = 0.7V, β = 200. |  | CO3 | L3 |
| OR |  |  |  |  |  |
| 7 | a) | Draw and explain the characteristics of PNP transistor in CE configuration | 5 | CO3 | L2 |
|  | b) | Given an NPN transistor for which α= 0.98, ICO = 2μA, and ICEO=16μA. A common-emitter connection is used as shown in below Fig. with VCC = 12V and RC = 4KΩ. What is the minimum base current required in order for the transistor to enter into saturation region? | 5 | CO3 | L3 |
| UNIT-IV |  |  |  |  |  |
| 8 |  | With neat sketches, explain the construction, principle of operation, and characteristics of an n-channel JFET. | 5 | CO4 | L2 |
|  |  | The N-channel JFET has IDSS = 20mA, VP = -8V and gmo = 5000μs. Determine the values of trans conductance at VGS= -4V. | 5 | CO4 | L3 |
| OR |  |  |  |  |  |
| 9 | a) | Compare Junction Field Effect Transistor with Bipolar Junction Transistor | 5 | CO4 | L2 |
|  | b) | A FET amplifier in the common-source configuration uses a load resistance of 500 kΩ. The ac drain resistance of the device is 100 kΩ, and the transconductance is 0.8 mAV1. Calculate the voltage gain of the amplifier. | 5 | CO4 | L3 |
| UNIT-V |  |  |  |  |  |
| 10 | a) | Differentiate between Zener breakdown & avalanche breakdown and mention applications of the Zener diode. | 5 | CO5 | L2 |
|  | b) | With a neat sketch, explain the CMOS inverter with a truth table | 5 | CO5 | L2 |
| OR |  |  |  |  |  |
| 11 | a) | Explain the principle operation of the SCR? | 5 | CO5 | L2 |
|  | b) | Find the maximum and minimum current flowing through the Zener diode, for the Zener regulator shown in the following figure. | 5 | CO5 | L3 |

---oo0oo---

CO: Course Outcomes    BL Blooms Taxonomy Levels

Note:1. Font style : Bookman Old Style-Size-12
          2. Marks for sub question are to be mentioned in the question paper. The questions prepared                   
              shall be either (5M + 5M) or (10M) only. Other combinations are not acceptable.

3.	There shall not be morethan three 10M questions in the question paper

4. The Course Outcome (CO) and Bloom’s Taxonomy Level (BL) shall be mentioned for each question.