| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: A7ME01

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

I B.Tech I Semester Regular Examinations December-2025/January-2026

ENGINEERING MECHANICS

(AERO & MECH)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			5 x 2M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | State the parallelogram law of forces. | 2 | 1 | 1 |
|  | b) | Define coefficient of friction. | 2 | 2 | 2 |
|  | c) | Differentiate between centroid and centre of gravity. | 2 | 3 | 1 |
|  | d) | Define displacement, velocity & acceleration. | 2 | 4 | 1 |
|  | e) | State D’Alemberts principle. | 2 | 5 | 1 |

PART- B 			5 x 10M = 50Marks

|  | a) | State and prove Lami’s theorem. | 5 | 1 | 3 |
| --- | --- | --- | --- | --- | --- |
| 2 | b) | For the system of forces shown, Find the magnitude and direction of resultant | 5 | 1 | 3 |
| OR |  |  |  |  |  |
| 3 |  | Find the magnitude and direction of the resultant force. Also find the position of the resultant force from point P of the bar PS. | 10 | 1 | 3 |
| Page 1 of 3 |  |  |  |  |  |
| 4 |  | Determine the force P required to just start the block. Take coefficient of friction=0.5 | 10 | 2 | 4 |
| OR |  |  |  |  |  |
| 5 | a) | What is the least value of P to just cause motion of the system shown in figure towards the right? Assume coefficient of friction to be 0.25. Body A weighs 650 N and θ=300 | 5 | 2 | 3 |
|  | b) | Define friction? What are the laws of friction | 5 | 2 | 2 |
| 6 | a) | Find the centroid of the shaded area shown in the figure. | 5 | 3 | 3 |
|  | b) | A corner of radius 20 mm is cut off from a square plate of 40 mm side as shown in the figure. Find the centre of gravity of the remaining plate about base. | 5 | 3 | 3 |
| OR |  |  |  |  |  |
| 7 |  | Find the moment of inertia of a T-section with flange as 150 mm × 50 mm and web as 150 mm × 50 mm about X-X and Y-Y axes through the centre of gravity of the section. | 10 | 3 | 3 |
| Page 2 of 3 |  |  |  |  |  |
| 8 |  | The motion of a particle is given by the equation s=2t4-16t3+2t2 where s is in meter and t in seconds. Compute the values of velocity and acceleration when t=2 sec. | 10 | 4 | 3 |
| OR |  |  |  |  |  |
| 9 | a) | Derive the expression for Impulse-Momentum equation. | 5 | 4 | 2 |
|  | b) | A particle under a constant deceleration is moving in a straight line and covers a distance of 40m in first 2 seconds and 60m in the next 5 seconds. Calculate distance it covers in the next subsequent 4 seconds and the total distance covered before it comes to rest. | 5 | 4 | 3 |
| 10 |  | Two bodies A and B of mass 80 kg and 20 kg are connected by a thread and move along a rough horizontal plane under the action of a force 400 N applied to the first body of mass 80 kg as shown in Figure. The coefficient of friction between the sliding surfaces of the bodies and the plane is 0.3. Determine the acceleration of the two bodies and the tension in the thread, using D' Alembert’s principle. | 10 | 5 | 4 |
| OR |  |  |  |  |  |
| 11 | a) | After the block in the figure has moved 3 m from rest by the constant force P. Find the velocity of the block. Angle of the plan =300 | 5 | 5 | 3 |
|  | b) | Derive work-energy equation for translation. | 5 | 5 | 2 |

---oo0oo---

Page 3 of 3