| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: A7EE06

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

I B.Tech I Semester Regular Examinations December-2025/January-2026

INTRODUCTION TO ELECTRICAL ENGINEERING

(ECE)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			5 x 2M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | What is the drop across the 150 Ω resistor in Fig. | 2 | 1 | 3 |
|  | b) | Define R.M.S. value of an alternating quantity | 2 | 2 | 2 |
|  | c) | What is transformation ratio in single phase transformer? | 2 | 3 | 1 |
|  | d) | Draw the torque – slip characteristics of an Induction motor | 2 | 4 | 2 |
|  | e) | What are the advantages of Power factor improvement | 2 | 5 | 2 |

PART- B 			5 x 10M = 50Marks

| 2 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  |  | Using Norton’s theorem, calculate the current in the 5 Ω resistor in the circuit shown in Fig. | 10 | 1 | 3 |
| OR |  |  |  |  |  |
| 3 |  | Calculate the current ‘I’ by using superposition theorem | 10 | 1 | 3 |
| 4 | a) | A 3-phase, 400-V (line voltage), balanced supply is connected to a load whose impedance per phase is 20 Ω. If Load connected in Star (Y), Calculate: The phase voltage, the phase current and the line current | 5 | 2 | 3 |
|  | Page 1 of 2 |  |  |  |  |
|  | b) | Explain the concept of resonance in a series R–L–C circuit. Derive the expression for resonant frequency and discuss the conditions under which resonance occurs. | 5 | 2 | 2 |
| OR |  |  |  |  |  |
| 5 | a) | A single-phase AC supply of 230 V, 50 Hz is connected to a series R–L–C circuit consisting of a 20 Ω resistor, a 0.15 H inductor, and a 40 μF capacitor. Calculate: The inductive reactance, The capacitive reactance, The net impedance of the circuit, the circuit current, The power factor (PF) of the circuit | 5 | 2 | 3 |
|  | b) | A choke coil is connected in series with a non inductive resistor. The combination is connected across 250V, 50Hz supply and draws a current of 5 A from the line. The voltage across the choke coil and the resistor are 125V and 200V respectively. Calculate the resistance and reactance of the coil, | 5 | 2 | 4 |
| 6 |  | Explain the working principle of single phase transformer and derive the expression for its EMF equation | 10 | 3 | 2 |
| OR |  |  |  |  |  |
| 7 | a) | Describe the major losses in a transformer and explain the factors that contribute to these losses | 5 | 3 | 2 |
|  | b) | What is the significance of equivalent circuit of transformer and explain the procedure | 5 | 3 | 2 |
| 8 |  | Describe the operating principle of a 3-phase induction motor, and derive the torque equation of 3-phase induction motor. | 10 | 4 | 2 |
| OR |  |  |  |  |  |
| 9 | a) | Explain the performance characteristics of a DC shunt machine and describe its common applications. | 5 | 4 | 2 |
|  | b) | Describe the basic operating principle of a synchronous generator and how voltage is induced in its armature | 5 | 4 | 3 |
| 10 |  | Explain about the SFU and ELCB with suitable diagrams. | 10 | 5 | 2 |
| OR |  |  |  |  |  |
| 11 | a) | Explain how a battery backup system functions to provide uninterrupted power when the main supply fails | 5 | 5 | 2 |
|  | b) | In an apartment, the occupants use various electrical appliances daily, including two 9-W LED lamps used for 5 hours each, a 60-W table fan running for 7 hours, a 250-W television used for 4 hours, and a 1000-W microwave oven operated for 20 minutes. Using this information, calculate the total daily energy consumption of the apartment in kWh, determine the monthly energy usage for 30 days, and estimate the monthly electricity bill if the tariff rate is ₹7.00 per kWh. | 5 | 5 | 3 |

---oo0oo---

.	Page 2 of 2