| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: A7EC03

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

I B.Tech I Semester Regular Examinations December-2025/January-2026

ELECTRONIC DEVICES AND APPLICATIONS

(CSE)

Time: 3 Hours.								            Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			5 x 2M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | List the differences between an ideal diode and a practical diode | 2 | 1 | 1 |
|  | b) | Explain the clamping circuits. | 2 | 2 | 2 |
|  | c) | Define the current gain of the common emitter configuration. | 2 | 3 | 1 |
|  | d) | Why N-channel FETs are preferred over P-channel FETs. | 2 | 4 | 3 |
|  | e) | List the applications of the Zener diode. | 2 | 5 | 2 |

PART- B 			5 x 10M = 50Marks

| 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | M | CO | BL |
|  | a) | Explain the operation of the PN Junction Diode and draw the V-I Characteristics of the Ge-Diode and Si-Diode with Forward Bias and Reverse Bias. | 5 | 1 | 2 |
|  | b) | Calculate the value of dc resistance and ac resistance of a germanium junction diode at 25°C with Io = 25 µA and at an applied voltage of 0.2 V across the diode. | 5 | 1 | 3 |
| OR |  |  |  |  |  |
| 3 | a) | Derive the Diode Current Equation? | 5 | 1 | 3 |
|  | b) | Calculate the diode current. The voltage across a silicon diode at room temperature (300 K) is 0.7 volts when 2 mA flows through it. If the voltage increases to 0.75 V, (assume VT = 26mv). | 5 | 1 | 3 |
| 4 | a) | Explain the working operation of the Bridge Rectifier with input & output forms and mention the advantages & applications. | 5 | 2 | 2 |
|  | b) | Find (i) Vdc (ii) Idc, and (iii) PIV, in a Fullwave rectifier. The transformer is connected to 200V, 60 Hz mains and the turns ratio of the step-down transformer is 11:1, assuming the diode is ideal. | 5 | 2 | 3 |
| OR Page 1 of 2 |  |  |  |  |  |
| 5 | a) | Explain the negative peak clipper with and without reference voltage. | 5 | 2 | 2 |
|  | b) | Define a clipper circuit? Sketch the output voltage waveform for the circuit shown in the following figure. | 5 | 2 | 3 |
| 6 | a) | Explain the CB configuration circuit, its output characteristics and indicate different regions. | 5 | 3 | 2 |
|  | b) | A transistor has IB= 100µA and IC = 2mA. Find. i) β of the transistor ii) α of the transistor, iii) Current IE, iv) If IB changes by +25µA and IC changes by +0.6mA, find the new value of β | 5 | 3 | 3 |
| OR |  |  |  |  |  |
| 7 | a) | Explain the operation of the Common Emitter Configuration and draw the input and output characteristics of a transistor in Common Emitter configuration. | 5 | 3 | 2 |
|  | b) | b) Determine IC, IE and α for a transistor circuit having IB = 15 µA and β = 150. | 5 | 3 | 3 |
| 8 | a) | Explain the expression for the pinch-off voltage VP​, of an n-channel JFET. | 5 | 4 | 2 |
|  | b) | A FET amplifier in the common-source configuration uses a load resistance of 500KΩ. The ac drain resistance of the device is 100KΩ, and the trans conductance is 0.8mAV –1. Solve the voltage gain of the amplifier | 5 | 4 | 3 |
| OR |  |  |  |  |  |
| 9 |  | Explain the operation of enhancement-mode and depletion-mode MOSFETs. | 10 | 4 | 2 |
| 10 | a) | Explain the principle of operation of a Silicon Controlled Rectifier (SCR) using appropriate waveforms. | 5 | 5 | 2 |
|  | b) | Explain the construction and operation of an LED with the help of a neat sketch. | 5 | 5 | 2 |
| OR |  |  |  |  |  |
| 11 | a) | Compare CMOS and FinFET technologies based on their basic structure and operation. | 5 | 5 | 3 |
|  | b) | Explain the construction and working principle of a solar cell with a neat sketch. | 5 | 5 | 2 |

---oo0oo---

Page 2 of 2