| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MLR20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: A5AE06

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

II B.Tech I Semester Supplementary Examination December-2025

MECHANICS OF FLUIDS

(AERO)

Time: 3 Hours.								Max. Marks: 70

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 20 marks. Answer all Questions in part A.

3. Part -B consists of 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 		10 x 2M = 20Marks

| 1 | a) | What is the difference between absolute pressure and gauge pressure? | 2M | CO1 | L2 |
| --- | --- | --- | --- | --- | --- |
|  | b) | State hydrostatic law and mention its mathematical form. | 2M | CO1 | L1 |
|  | c) | Write the incompressible Bernoulli’s equations and mention its components | 2M | CO2 | L1 |
|  | d) | Define steady and unsteady flows. | 2M | CO2 | L1 |
|  | e) | What are minor losses? | 2M | CO3 | L1 |
|  | f) | What are the repeating variables in Buckingham π theorem? | 2M | CO3 | L1 |
|  | g) | What is a shock wave? | 2M | CO4 | L1 |
|  | h) | State the criteria for transition of boundary layer. | 2M | CO4 | L1 |
|  | i) | Define Mach number. Write its mathematical from as a function of speed of sound. | 2M | CO5 | L1 |
|  | j) | When the flow properties are said to be stagnation properties? | 2M | CO5 | L1 |

PART- B 		5 x 10M = 50Marks

| 2 | a) | Classify manometers. Describe the working of a simple U tube manometer. | 5M | CO1 | L2 |
| --- | --- | --- | --- | --- | --- |
|  | b) | A rectangular plate (3 m × 2 m) is submerged vertically in water such that the top edge is 1 m below the free surface. Determine (i) Total hydrostatic force on the plate and (ii) Location of the centre of pressure | 5M | CO1 | L3 |
| OR |  |  |  |  |  |
| 3 | a) | Define viscosity and distinguish between dynamic and kinematic viscosity. | 5M | CO1 | L2 |
|  | b) | A plate of area 0.3 m² is pulled parallel to another plate with a velocity of 1 m/s. The gap between the plates is 2 mm, and the fluid has a dynamic viscosity of 0.8 Ns/m². Compute the shear stress and force required to move the plate. | 5M | CO1 | L3 |
| 4 | a) | Define and differentiate streamline, pathline, and streakline with neat illustrations. | 5M | CO2 | L1 |
|  | Page 1 of 2 |  |  |  |  |
|  | b) | A flow is given by velocity components: U = 4x and v = -4y. Determine whether the flow is incompressible and whether it is irrotational. | 5M | CO2 | L3 |
| OR |  |  |  |  |  |
| 5 | a) | Define vortex motion. Discuss about forced vortex motion and free vortex motion. | 5M | CO2 | L2 |
|  | b) | Water flows through a 1 m diameter pipe at 3 m/s and enters a 0.5 m diameter pipe. Find the velocity in the smaller pipe using the continuity equation. | 5M | CO2 | L3 |
| 6 |  | Derive Darcy–Weisbach equation for head loss due to friction in pipes. State all assumptions clearly. | 10M | CO3 | L3 |
| OR |  |  |  |  |  |
| 7 | a) | Explain the working of a Pitot tube with a neat sketch. | 5M | CO3 | L2 |
|  | b) | Water flows over a rectangular notch 0.6 m wide with a head of 300 mm. Find the discharge. | 5M | CO3 | L2 |
| 8 | a) | Explain the key thickness of boundary layer Displacement thickness, Momentum thickness. | 5M | CO4 | L2 |
|  | b) | What is boundary layer transition? Explain the factors influencing transition from laminar to turbulent layer. | 5M | CO4 | L2 |
| OR |  |  |  |  |  |
| 9 |  | A viscous fluid flows over a flat plate such that the boundary layer thickness at a distance 1.3 m from the leading edge is 12 mm. Assuming the flow to be laminar, determine the boundary layer thickness at a distance of (a) 0.2 m, (b) 2.0 m, and (c) 20 m from the leading edge. | 10M | CO4 | L3 |
| 10 | a) | Define compressible flow. Distinguish between compressible and incompressible flows with examples. | 5M | CO5 | L2 |
|  | b) | A airplane has attained a speed of M=2.0 at an altitude of 16 km, where the temperature is approximately -560C. Assuming γ=1.4 and R=290 J/kg K, determine speed of the airplane in kilometre per hour (kmph). | 5M | CO5 | L3 |
| OR |  |  |  |  |  |
| 11 | a) | Classify flows based on Mach number and give practical examples. | 5M | CO5 | L2 |
|  | b) | A supersonic flow has Mach number 2.5 and static temperature 240 K. Calculate (i) Speed of sound (ii) Velocity of flow (iii) Stagnation temperature | 5M | CO5 | L3 |

---oo0oo---

Page 2 of 2