| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	A6EC05

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

II B.Tech I Semester Regular/Supplementary Examinations December-2025

ANALOG CIRCUITS

(ECE)

Time: 3 Hours.								              Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			  10 x 1M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | State Millers Theorem. | 1 | 1 | 1 |
|  | b) | Draw RC coupled amplifier circuit diagram. | 1 | 1 | 2 |
|  | c) | Draw the circuit of the hybrid-π model CE amplifier. | 1 | 2 | 2 |
|  | d) | Classify power amplifiers. | 1 | 2 | 2 |
|  | e) | What is the condition for Barkhausen criterion? | 1 | 3 | 2 |
|  | f) | Define Oscillator? | 1 | 3 | 3 |
|  | g) | Draw the equation of low pass RC circuit as Integrator. | 1 | 4 | 2 |
|  | h) | Define Clipper? | 1 | 4 | 3 |
|  | i) | Define stable state and Quasi Stable State? | 1 | 5 | 3 |
|  | j) | Draw the circuit diagram of a Schmitt trigger. | 1 | 5 | 2 |

PART- B 			  5 x 10M = 50Marks

| 2 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Discuss the analysis of common emitter amplifier with emitter resistance using h-parameter model? | 5 | 1 | 3 |
|  | b) | Explain the working principle of RC coupled amplifier. | 5 | 1 | 2 |
| OR |  |  |  |  |  |
| 3 | a) | Classify amplifiers. | 5 | 1 | 2 |
|  | b) | Explain the working principle of Darlington pair. | 5 | 1 | 3 |
| 4 | a) | Derive the expression for efficiency of a class A power amplifier. | 5 | 2 | 4 |
|  | b) | Draw the complimentary-symmetry class-B power amplifier and derive the efficiency. | 5 | 2 | 2 |
| OR Page 1 of 2 |  |  |  |  |  |
| 5 | a) | Discuss about harmonic distortion | 5 | 2 | 3 |
|  | b) | Draw the Hybrid Pi model of CE Amplifier and label the parameters. | 5 | 2 | 2 |
| 6 |  | Derive the expression for frequency of oscillations for Colpitts oscillator. | 10 | 3 | 4 |
| OR |  |  |  |  |  |
| 7 | a) | In an Hartley oscillator, if L1=0.2mH, L2=0.3mH and C=0.003 µF, calculate the frequency of its oscillation | 5 | 3 | 3 |
|  | b) | Draw the circuit diagram of current shunt feedback amplifier and derive expressions for gain. | 5 | 3 | 2 |
| 8 |  | Explain how a high-pass circuit acts as differentiator. | 10 | 4 | 3 |
| OR |  |  |  |  |  |
| 9 | a) | A pulse input is applied to low-pass RC circuit. Derive the expression for output wave form? | 5 | 4 | 4 |
|  | b) | Classify different types of clipper circuits. Draw the input output wave forms. | 5 | 4 | 2 |
| 10 | a) | Explain the triggering method of Monostable multivibrator | 5 | 5 | 3 |
|  | b) | Explain the operation of a Schmitt Trigger and derive the expressions for UTP and LTP. | 5 | 5 | 3 |
| OR |  |  |  |  |  |
| 11 | a) | Draw the circuit of a miller integrator and explain how it improves the linearity of the sweep waveform? | 5 | 5 | 3 |
|  | b) | Explain the operation of a transistor based bootstrap sweep generator and derive the expression for Sweep time. | 5 | 5 | 3 |

---oo0oo---

Page 2 of 2