| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: A6AE05

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

II B.Tech I Semester Regular/Supplementary Examinations December-2025

AERO THERMODYNAMICS

(AERO)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 1M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | What is meant by thermodynamics? | 1 | 1 | 1 |
|  | b) | Define specific heat capacity at constant Pressure. | 1 | 1 | 1 |
|  | c) | State the Kelvin–Planck statement of second law of Thermodynamics | 1 | 2 | 2 |
|  | d) | What is difference between heat pump and refrigeration? | 1 | 2 | 1 |
|  | e) | What is saturation temperature and saturation pressure? | 1 | 3 | 1 |
|  | f) | Draw T–S diagram for the Rankine cycle | 1 | 3 | 1 |
|  | g) | State the purpose of velocity triangles in compressor analysis. | 1 | 4 | 1 |
|  | h) | Write the applications of axial flow compressors? | 1 | 4 | 1 |
|  | i) | Why is turbine cooling necessary in modern gas turbines? | 1 | 5 | 1 |
|  | j) | Define degree of reaction in turbine design. | 1 | 5 | 1 |

PART- B 			5 x 10M = 50Marks

| 2 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  |  | Air at a temperature of 15°C passes through a heat exchanger at a velocity of 30 m/s where its temperature is raised to 800°C. It then enters a turbine with the same velocity of 30 m/s and expands until the temperature falls to 650°C. On leaving the turbine, the air is taken at a velocity of 60 m/s to a nozzle where it expands until the temperature has fallen to 500°C. If the air flow rate is 2 kg/s, calculate a) the rate of heat transfer to the air in the heat exchanger b) the power output from the turbine assuming no heat loss and c) the velocity at the nozzle exit, assuming zero heat loss. Take enthalpy of air as h = Cp.T where Cp is the specific heat = 1.005 kJ/kg K and T is the temperature. | 10 | 1 | 3 |
| OR |  |  |  |  |  |
| 3 | a) | Explain the following with examples: Path Function, Point Function, Thermodynamic Cycle | 5 | 1 | 2 |
|  | Page 1 of 2 |  |  |  |  |
|  | b) | What is a Quasi-Static process? Explain with a neat diagram. | 5 | 1 | 2 |
| 4 |  | A house hold refrigerator is maintained at a temperature of 275K. Every time the door is opened, warm material is placed inside, introducing an average of 420 kJ, but making only small changes in the temperature of the refrigerator. The door is opened 20 times a day, and the refrigerator operates at 15% of the ideal COP. The cost of work is 2.50 per kW/hr. What is the bill for the month of April for this refrigerator? The atmosphere is at 303K. | 10 | 2 | 2 |
| OR |  |  |  |  |  |
| 5 | a) | Explain how the Second Law of Thermodynamics rules out PMM of the second kind. | 5 | 2 | 3 |
|  | b) | A refrigerator extracts 200 kJ of heat from a cold reservoir at 270 K and rejects it to a reservoir at 300 K. Calculate the COP. | 5 | 2 | 3 |
| 6 |  | In a steam turbine, steam at 20 bar, 360° C is expanded to 0.08 bar. It then enters a condenser, where it is condensed to saturated liquid water. The pump feeds back the water into the boiler. Assuming ideal processes, determine per kg of steam the network and the cycle efficiency | 10 | 3 | 4 |
| OR |  |  |  |  |  |
| 7 | a) | A gas turbine plant operates on the Brayton cycle between Tmin = 300 K and Tmax = 1073 K. The pressure ratio of the Brayton cycle is given as 6. Find the maximum work done per kg of air and the corresponding cycle efficiency. Compare the efficiency with carnot efficiency operating between the same two temperature. | 5 | 3 | 3 |
|  | b) | In an air standard otto cycle the compression ratio is 6. The compression begins 400C and 0.1 MPa. The heat supplied per kg of air at the end of the compression is 852 KJ. Find the temperature and pressure at all salient points and the efficiency. | 5 | 3 | 3 |
| 8 |  | Derive the elementary theory of axial flow compressors using Euler theory and explain the assumptions made. | 10 | 4 | 3 |
| OR |  |  |  |  |  |
| 9 | a) | With a suitable sketch explain the working principle of axial flow compressor. | 5 | 4 | 2 |
|  | b) | Explain the design considerations for compressor blades | 5 | 4 | 2 |
| 10 | a) | Explain and how multi-staging improves efficiency by calculating the work output for a two-stage turbine compared to a single-stage turbine with the same pressure ratio. | 5 | 5 | 3 |
|  | b) | Explain various methods to reduce stage losses in turbines. | 5 | 5 | 3 |
| OR |  |  |  |  |  |
| 11 | a) | Briefly discuss the methods of turbine blade cooling and mention its advantages and disadvantages. | 5 | 5 | 3 |
|  | b) | Explain how blade profile design affects turbine efficiency and losses. | 5 | 5 | 3 |

---oo0oo---					 Page 2 of 2