| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	A6EE04

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

II B.Tech I Semester Regular/Supplementary Examinations December-2025

ELECTRICAL CIRCUITS-II

(EEE)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 1M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Determine the co-efficient of coupling for two coils having L1 = 2 H, L2 = 8 H and M = 3 H? | 1 | 1 | 2 |
|  | b) | Define self-inductance. | 1 | 1 | 1 |
|  | c) | Write the relation between line and phase currents in delta connection? | 1 | 2 | 1 |
|  | d) | What is a three-phase four-wire system? | 1 | 2 | 1 |
|  | e) | What is the Laplace Transform of a unit step function u(t)? | 1 | 3 | 1 |
|  | f) | What is the Laplace Transform of eat? | 1 | 3 | 1 |
|  | g) | What are initial conditions in RL circuits? | 1 | 4 | 1 |
|  | h) | What is the time constant of an RC circuit? | 1 | 4 | 1 |
|  | i) | Define the cut-off frequency? | 1 | 5 | 1 |
|  | j) | Write the condition for characteristic impedance of a constant-k low-pass filter? | 1 | 5 | 1 |

PART- B 			5 x 10M = 50Marks

| 2 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Derive the relation between K, L, M in the analysis of coupled circuits? | 5 | 1 | 2 |
|  | b) | State and explain Faraday’s Laws of Electro Magnetic Induction? | 5 | 1 | 2 |
| OR |  |  |  |  |  |
| 3 | a) | Derive the expression for equivalent inductance when the coupled inductors are connected in Series aiding and series subtracting? | 5 | 1 | 3 |
|  | b) | Two coupled coils have a self-inductances 37.5 mH and 193 mH, with the mutual inductance of 63.75mH. Find the co efficient of coupling? | 5 | 1 | 5 |
| Page 1 of 2 |  |  |  |  |  |
| 4 | a) | Write the relationship between Phase and Line voltages, currents in star and delta connected circuits? | 5 | 2 | 3 |
|  | b) | Explain the measurement of three-phase power using two wattmeter? | 5 | 2 | 2 |
| OR |  |  |  |  |  |
| 5 | a) | The power input to a three-phase induction motor is read by two wattmeters. The readings are 1000 W and 500 W. Find out the pf of the motor. If the line voltage is 400 V, find the line current? | 5 | 2 | 5 |
|  | b) | Explain the significance of phase sequence? | 5 | 2 | 2 |
| 6 |  | Analyse parallel RC circuit using Laplace transform for ramp and impulse inputs? | 10 | 3 | 4 |
| OR |  |  |  |  |  |
| 7 | a) | Explain the basic properties of Laplace transforms? | 5 | 3 | 2 |
|  | b) | Analyse series RL circuit using Laplace transform for step input? | 5 | 3 | 3 |
| 8 |  | Derive the transient response of an RLC circuit with DC excitation? | 10 | 4 | 4 |
| OR |  |  |  |  |  |
| 9 | a) | Analyse the transient response of an RC series circuit for DC excitation and find i(t)? | 5 | 4 | 5 |
|  | b) | Analyse the transient response of an RL series circuit for DC excitation and find i(t)? | 5 | 4 | 3 |
| 10 |  | List the different types of filters and explain in detail? | 10 | 5 | 2 |
| OR |  |  |  |  |  |
| 11 | a) | Define Band Elimination Filter. Derive the operational characteristics of a band elimination filter? | 5 | 5 | 2 |
|  | b) | Explain in detail the constant K type low pass filters? | 5 | 5 | 2 |

---oo0oo---

Page 2 of 2