| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MLR20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	A5EE05

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

II B. Tech I Semester Supplementary Examination, December 2025

ELECTRICAL CIRCUIT ANALYSIS

(EEE)

Time: 3 Hours.								Max. Marks: 70

Note: 1. This question paper contains two parts, A and B.

2. Part A is Compulsory, which carries 20 marks. Answer all Questions in part A.

3. Part B consists of 5units. Answer any one question from each unit. Each question 
              carries 10 marks and may have A, B, C as sub-questions.

PART- A 			10 x 2M = 20Marks

| 1 | a) | State superposition theorem. | 2M | CO1 | L1 |
| --- | --- | --- | --- | --- | --- |
|  | b) | Mention any two differences between the tie-set and cut-set matrices. | 2M | CO1 | L1 |
|  | c) | List any two applications of the two-port network parameters. | 2M | CO2 | L1 |
|  | d) | Define impedance (Z) parameters of a two-port network. | 2M | CO2 | L1 |
|  | e) | Write the differential equation for the transient current in an RL series circuit. | 2M | CO3 | L2 |
|  | f) | State the condition for overdamped response in an RLC circuit. | 2M | CO3 | L1 |
|  | g) | What is the dot convention in coupled circuits? | 2M | CO4 | L1 |
|  | h) | Draw the locus of impedance for a series RL circuit. | 2M | CO4 | L3 |
|  | i) | Define a balanced three-phase system. | 2M | CO5 | L1 |
|  | j) | What is meant by the shifting of neutral in unbalanced 3-phase systems? | 2M | CO5 | L2 |

PART- B 			5 x 10M = 50Marks

| 2 | a) | Derive the condition for maximum power transfer to the load? | 5M | CO1 | L4 |
| --- | --- | --- | --- | --- | --- |
|  | b) | State and explain Millimans theorem. | 5M | CO1 | L3 |
| OR |  |  |  |  |  |
| 3 |  | With An example, explain how to form a reduced incidence matrix and state its properties. | 10M | CO1 | L3 |
| 4 |  | Write the expressions for Y-parameters of a two-port network and derive the conditions under which the network becomes symmetric in terms of Y-parameters. | 10M | CO2 | L4 |
| OR |  |  |  |  |  |
| 5 | a) | Discuss the Z parameters and derive the equation for a series connection of two two-port networks. | 5M | CO2 | L4 |
|  | Page 1 of 2 |  |  |  |  |
|  | b) | Obtain ABCD parameters for the network shown | 5M | CO2 | L3 |
| 6 | a) | Explain the transient response of a series RL circuit with DC excitation using a differential equation approach. | 5M | CO3 | L4 |
|  | b) | For the network shown in Fig., the switch is closed at t = 0, determine v, dv/dt, and d2v/ dt2 at t = 0+. | 5M | CO3 | L3 |
| OR |  |  |  |  |  |
| 7 | a) | Describe the effect of initial capacitor voltage and initial inductor current on the transient analysis. Give examples. | 5M | CO3 | L3 |
|  | b) | In the network shown in Fig., the switch is closed at t = 0, the steady-state being reached before t = 0. Determine the current through the inductor of 3 H. | 5M | CO3 | L3 |
| 8 | a) | Discuss the behaviour of two inductors connected in series with mutual coupling. | 5M | CO4 | L2 |
|  | b) | In a parallel resonance circuit (Tank circuit) R=2Ω, L=1mH, and C=10μF, find the Resonant frequency, Dynamic impedance, and Bandwidth. | 5M | CO4 | L3 |
| OR |  |  |  |  |  |
| 9 |  | Derive the relation between K, L, M in magnetically coupled circuits. | 10M | CO4 | L4 |
| 10 | a) | Write the relationship between voltage and current in a star and delta circuits. | 5M | CO5 | L4 |
|  | b) | A balanced star-connected load having an impedance (15+j20) Ω per phase is connected to a three-phase 440 V,50Hz supply. Find line currents and phase voltages. Assume the RYB phase sequence and also calculate the power drawn by the load. | 5M | CO5 | L3 |
| OR |  |  |  |  |  |
| 11 | a) | Derive the formula for power measurement using the two-wattmeter method, and explain how power factor is determined. | 5M | CO5 | L4 |
|  | b) | A 400V,3¢ supply feeds an unbalanced 3-wire star-connected load. The branch impedances of the load are ZR= (4+j8)Ω ,ZY=(3+j4)Ω ,ZB=(5+j20)Ω. Find the line currents and voltages across the phase impedance. Assume RYB phase sequence. | 5M | CO5 | L3 |

---oo0oo--- 					Page 1 of 2