| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MLR20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: A5EE06

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

II B.Tech I Semester Supplementary Examinations December-2025

ELECTRICAL MACHINES-1

(EEE)

Time: 3 Hours.								Max. Marks: 70

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 20 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 2M = 20Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Write the Principle of electrical generator? | 2 | 1 | 1 |
|  | b) | Name the applications of D.C generators. | 2 | 1 | 1 |
|  | c) | What is back e.m.f? | 2 | 2 | 1 |
|  | d) | What is meant by a compound motor? | 2 | 2 | 1 |
|  | e) | What are the different types of losses in D.C machines? | 2 | 3 | 1 |
|  | f) | What is a fields test? | 2 | 3 | 1 |
|  | g) | Draw the phasor diagram of transformer with inductive load. | 2 | 4 | 1 |
|  | h) | Define a transformer. | 2 | 4 | 1 |
|  | i) | What is parallel operation in three phase transformers? | 2 | 5 | 1 |
|  | j) | What are the different types of tap changers? | 2 | 5 | 1 |

PART- B 			5 x 10M = 50Marks

| 2 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Derive the e.m.f. equation of a D.C generator. | 5 | 1 | 2 |
|  | b) | A 8-pole lap wound armature rotated at 350 r.p.m is required to generate 260 V. The useful flux per pole is about 0.05 Wb. If the armature has 120 conductors. Determine the induced EMF. | 5 | 1 | 3 |
| OR |  |  |  |  |  |
| 3 | a) | Explain the different types of generators. | 5 | 1 | 2 |
|  | b) | An 6 pole lap wound generator armature has 720 conductors, a flux of 30 mWb and a speed of 600 r.p.m. Calculate the e.m.f. generated on open circuit. | 5 | 1 | 3 |
| Page 1 of 2 |  |  |  |  |  |
| 4 |  | Explain the characteristics of D.C motor. | 10 | 2 | 2 |
| OR |  |  |  |  |  |
| 5 | a) | Derive an expression for the torque of a D.C motor. | 5 | 2 | 2 |
|  | b) | A D.C series motor is rated 230 V ,1000 r.p.m ,80 A .The series field resistance is 0.11Ω and the armature resistance is 0.14Ω .If the flux at an armature current of 20 A. Calculate the back emf? | 5 | 2 | 3 |
| 6 |  | What is the necessity of a starter? Explain 3-point starter with neat diagram. | 10 | 3 | 2 |
| OR |  |  |  |  |  |
| 7 | a) | Explain Hopkinson’s test and corresponding circuit diagram. | 5 | 3 | 2 |
|  | b) | A 440 V D.C shunt motor takes a no load current of 2.5 A. The resistance of the shunt field and the armature are 550 Ω and 1.2 Ω respectively. The full load line current is 32 A. Find the full load output and the efficiency of the motor. | 5 | 3 | 3 |
| 8 |  | Explain in detail about open circuit test and short circuit test of a transformer with a neat circuit diagram. | 10 | 4 | 2 |
| OR |  |  |  |  |  |
| 9 | a) | Derive the e.m.f equation of a single-phase transformer. | 5 | 4 | 2 |
|  | b) | A 11000/400V distribution transformer takes a no load current of 1A at a power factor of 0.24 lagging find i) ore loss current ii) magnetizing current and iii) iron loss. | 5 | 4 | 3 |
| 10 | a) | Explain the principle of operation of an auto transformer. | 5 | 5 | 2 |
|  | b) | Explain parallel operation of a 3-phase transformer. | 5 | 5 | 3 |
| OR |  |  |  |  |  |
| 11 |  | What are the various types of three-phase transformers? Discuss their merits and demerits? | 10 | 5 | 2 |

---oo0oo---

Page 2 of 2