| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MLR20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	A5EE08

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

II B.Tech I Semester Supplementary Examination December-2025

ELECTRICAL MEASUREMENTS AND INSTRUMENTATION

(EEE)

Time: 3 Hours.								Max. Marks: 70

Note: 1. This question paper contains two parts, A and B.

2. Part A is Compulsory, which carries 20 marks. Answer all Questions in part A.

3. Part B consists of 5units. Answer any one question from each unit. Each question 
              carries 10 marks and may have A, B, C as sub-questions.

PART- A 			10 x 2M = 20Marks

| 1 | a) | State the principle of a PMMC ammeter. | 2M | CO1 | L1 |
| --- | --- | --- | --- | --- | --- |
|  | b) | What is the purpose of series resistance in a voltmeter? | 2M | CO1 | L2 |
|  | c) | State two applications of the Crompton potentiometer. | 2M | CO2 | L1 |
|  | d) | What is the function of the phase-shifting transformer in AC potentiometers? | 2M | CO2 | L2 |
|  | e) | State the working principle of a dynamometer-type wattmeter. | 2M | CO3 | L1 |
|  | f) | What is reactive power? Mention its unit. | 2M | CO3 | L1 |
|  | g) | What is the function of null detectors in bridge circuits? | 2M | CO4 | L2 |
|  | h) | What is the purpose of using a Wagner earthing device in AC bridges? | 2M | CO4 | L2 |
|  | i) | Define the gauge factor of a strain gauge. | 2M | CO5 | L1 |
|  | j) | Mention one feature of a digital storage oscilloscope (DSO). | 2M | CO5 | L1 |

PART- B 			5 x 10M = 50Marks

| 2 | a) | Classify measuring instruments and explain the functions of deflecting, damping, and controlling torques. | 5M |  | CO1 | L2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | b) | Discuss the different types of errors occurring in electrical measurements and their compensation methods. | 5M |  | CO1 | L3 |  |
| OR |  |  |  |  |  |  |  |
| 3 | a) | Describe the role of shunts and multipliers for extending the range of ammeters and voltmeters. | 5M | CO1 |  |  | L2 |
|  | b) | For a moving-coil ammeter with internal resistance 0.5 Ω and full-scale current 5 A, determine the shunt needed to extend the range to 30 A. | 5M | CO1 |  |  | L3 |
| 4 | a) | Explain the construction and working principle of a Crompton DC potentiometer with a neat diagram. | 5M | CO2 |  |  | L2 |
|  | Page 1 of 2 |  |  |  |  |  |  |
|  | b) | A Crompton DC potentiometer is used to measure an unknown emf. When the standard cell of 1.0186 V is balanced at 101.86 cm, the unknown emf balances at 135 cm. Determine the unknown emf. | 5M | CO2 |  |  | L3 |
| OR |  |  |  |  |  |  |  |
| 5 | a) | Discuss the construction, working, and applications of potential transformers. | 5M | CO2 |  |  | L2 |
|  | b) | A PT (potential transformer) has a rated ratio of 11 kV / 110 V. If the actual secondary voltage is measured as 108.5 V when the primary is at 11 kV, compute the ratio error. | 5M | CO2 |  |  | L3 |
| 6 | a) | Distinguish between LPF (Low Power Factor) and UPF (Unity Power Factor) type wattmeter. | 5M | CO3 |  |  | L3 |
|  | b) | A dynamometer wattmeter reads 950 W when measuring a load. The instrument has a 0.5 A CT with a ratio of 20:1 and a PT with a ratio of 200:1. Find the actual power being consumed by the load. | 5M | CO3 |  |  | L2 |
| OR |  |  |  |  |  |  |  |
| 7 |  | Explain the two-wattmeter method of measuring active and reactive power in 3-phase systems. | 10M | CO3 |  |  | L2 |
| 8 | a) | Discuss the advantages and limitations of Desauty’s Bridge. | 5M | CO4 |  |  | L2 |
|  | b) | A capacitor of unknown value is connected in a Desauty’s bridge. The bridge has resistances R1 = 3 kΩ, R2 = 2 kΩ, R3 = 5 kΩ and known capacitor C2 = 1 µF. Calculate the value of unknown capacitance Cx at balance. | 5M | CO4 |  |  | L3 |
| OR |  |  |  |  |  |  |  |
| 9 | a) | Explain the working principle of Wein’s Bridge and its application in frequency measurement. | 5M | CO4 |  |  | L2 |
|  | b) | A Wien’s Bridge oscillator is tuned using R1 = R2 = 10 kΩ and C1 = C2 = 0.01 µF. Calculate the frequency of oscillation. | 5M | CO4 |  |  | L3 |
| 10 |  | Define a transducer. Classify various types of electrical transducers and explain the advantages of electrical transducers. | 10M | CO5 |  |  | L2 |
| OR |  |  |  |  |  |  |  |
| 11 |  | Explain the need and working of a time-base generator in a CRO. | 10M | CO5 |  |  | L2 |

---oo0oo---

Page 2 of 2