| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MLR20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	A5AE07

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

II B.Tech I Semester Supplementary Examination December-2025

ENGINEERING THERMODYNAMICS

(AERO)

Time: 3 Hours.								Max. Marks: 70

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 20 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 2M = 20Marks

| 1. | a) | Define the term thermodynamic system. | 2M | CO1 | L1 |
| --- | --- | --- | --- | --- | --- |
|  | b) | Give the applications of thermodynamics. | 2M | CO1 | L1 |
|  | c) | Define Internal Energy. | 2M | CO2 | L1 |
|  | d) | Define entropy. | 2M | CO2 | L1 |
|  | e) | What is an air standard cycle? | 2M | CO3 | L1 |
|  | f) | Discuss the advantages of Otto cycle. | 2M | CO3 | L1 |
|  | g) | Define specific heat. | 2M | CO4 | L1 |
|  | h) | State the term dryness fraction. | 2M | CO4 | L1 |
|  | i) | Define heat transfer. What are its positive and negative directions? | 2M | CO5 | L1 |
|  | j) | Describe pressure ratio in Brayton cycle. Give its expression. | 2M | CO5 | L1 |

PART- B 			5 x 10M = 50Marks

| 2 | a) | Illustrate quasi-static process and state its characteristic features. | 5M | CO1 | L2 |
| --- | --- | --- | --- | --- | --- |
|  | b) | A mass of gas is compressed in a quasi-static from 80 kPa, 0.1 m3 to 0.4 MPa, 0.03 m3 assuming the relationship between pressure and volume given as P𝑉n = 𝑐𝑜𝑛𝑠𝑡𝑎𝑛𝑡. Determine the work transfer for the process. | 5M | CO1 | L3 |
| OR |  |  |  |  |  |
| 3 |  | Discuss displacement work and derive the expression for Pdv work. | 10M | CO1 | L2 |
| 4 | a) | Explain about Carnot cycle with neat sketch. | 5M | CO2 | L2 |
|  | Page 1 of 2 |  |  |  |  |
|  | b) | A heat engine is supplied with 1130 kW of heat at constant temperature of 2920C and rejects heat at 500C. The following results were obtained. Determine whether the results report a reversible cycle or irreversible or impossible. (i) Heat rejected is 834 kW. (ii) If heat rejected is 556 kW. | 5M | CO2 | L3 |
| OR |  |  |  |  |  |
| 5 | a) | Describe the heat engine. With the help of suitable sketch. | 5M | CO2 | L1 |
|  | b) | Establish the change in entropy relations for Ideal gases. | 5M | CO2 | L2 |
| 6 |  | Derive the thermal efficiency expression for Diesel cycle. | 10M | CO3 | L3 |
| OR |  |  |  |  |  |
| 7 | a) | Compare the heat addition process for the Otto, Diesel and Dual cycles. Also, represent them on the P-V diagram. | 5M | CO3 | L2 |
|  | b) | A petrol engine works on air standard Otto cycle. The pressure and temperature of air at beginning is 1 bar, 1500C and compression ratio is 8. The heat supplied per kg of air is limited to 2000 kJ. Find air standard efficiency. | 5M | CO3 | L3 |
| 8 |  | Explain the working of Reheat Rankine cycle with suitable diagram. When is reheating of steam is recommended. | 10M | CO4 | L2 |
| OR |  |  |  |  |  |
| 9 | a) | Calculate volume, density, enthalpy and entropy of 3 kg of steam at 900C, having a dryness fraction of 0.92. | 5M | CO4 | L3 |
|  | b) | Discuss the p-v diagram of a pure substance with a neat sketch. | 5M | CO4 | L2 |
| 10 | a) | Illustrate the working of Brayton cycle with help of Pressure (P)-Volume (V) diagram and Temperature (T)- Entropy (S) diagrams. | 5M | CO5 | L2 |
|  | b) | What is the effect of regeneration on Brayton cycle efficiency? | 5M | CO5 | L2 |
| OR |  |  |  |  |  |
| 11 |  | In an ideal Brayton cycle, air from the atmosphere at 1 atm,300k is compressed to 6 atm and the maximum cycle temperature is limited to 1100K by using a large air-fuel ratio. If the heat supply is 100 MW. Find a) the thermal efficiency of the cycle b) work ratio c) power output. | 10M | CO5 | L3 |

---oo0oo---

Page 2 of 2