| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	A6EC06

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

II B.Tech I Semester Regular/Supplementary Examinations December-2025

SIGNALS AND SYSTEMS

(ECE)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 1M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Define the energy of a signal. | 1 | 1 | 1 |
|  | b) | What is meant by signal multiplication? | 1 | 1 | 1 |
|  | c) | Name any two properties of Fourier Series. | 1 | 2 | 1 |
|  | d) | Define inverse Fourier transform. | 1 | 2 | 1 |
|  | e) | What is system bandwidth? | 1 | 3 | 1 |
|  | f) | Explain the meaning of the impulse response of a system. | 1 | 3 | 1 |
|  | g) | What operation in the frequency domain corresponds to convolution in the time domain? | 1 | 4 | 1 |
|  | h) | How does convolution affect the response of LTI systems? | 1 | 4 | 1 |
|  | i) | What is the Z-transform of the unit step u[n]? | 1 | 5 | 1 |
|  | j) | Which transform is used for discrete-time LTI system analysis? | 1 | 5 | 1 |

PART- B 			5 x 10M = 50Marks

| 2 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Explain any three basic operations on signals: (a) Time shifting (b) Time reversal (c) Time scaling Illustrate each operation with suitable examples and sketches. | 5 | 1 | 2 |
|  | b) | xt=t for 0≤t≤1, and xt=0 elsewhere. Find the energy Eof x(t). | 5 | 1 | 3 |
| OR |  |  |  |  |  |
| 3 | a) | Explain periodic and non-periodic signals. Derive the condition for periodicity of continuous-time and discrete-time signals. | 5 | 1 | 3 |
|  | b) | Illustrate deterministic and non-deterministic signals. Give at least three examples from real-time systems. | 5 | 1 | 2 |
| Page 1 of 2 |  |  |  |  |  |
| 4 | a) | Clarify the Dirichlet’s conditions for the existence of a Fourier Series. Give suitable examples. | 5 | 2 | 2 |
|  | b) | Explain the convolution property of Fourier Transform. Prove that convolution in time domain corresponds to multiplication in frequency domain. | 5 | 2 | 4 |
| OR |  |  |  |  |  |
| 5 | a) | State and explain any three properties of Fourier Series. | 5 | 2 | 3 |
|  | b) | Derive the Fourier Transform of a double-sided exponential signal xt=e-a∣t∣, a>0. | 5 | 2 | 5 |
| 6 |  | Define the transfer function of an LTI system. Derive the relationship between transfer function, impulse response, and frequency response. | 10 | 3 | 4 |
| OR |  |  |  |  |  |
| 7 | a) | Discuss the effects of under-sampling. Explain aliasing and derive the relation between sampling frequency and signal bandwidth. | 5 | 3 | 3 |
|  | b) | A signal contains frequency components at 1 kHz, 3 kHz, and 5 kHz. It is sampled at fs = 6 kHz. (i) Find the aliased frequencies. (ii) Draw or list the spectrum after sampling. | 5 | 3 | 6 |
| 8 |  | Explain the concept of ROC in the Laplace Transform. Discuss how ROC determines stability and causality of a system. | 10 | 4 | 3 |
| OR |  |  |  |  |  |
| 9 | a) | Describe the graphical method of convolution with neat sketches and illustrate the steps involved | 5 | 4 | 2 |
|  | b) | Given the Fourier transforms: X(ω)=1jω+3,H(ω)=2jω+1 Find Yω=XωHω and determine yt by inverse Fourier transform. | 5 | 4 | 5 |
| 10 | a) | Discuss the significance of Z-transform in discrete-time signal and system analysis. | 5 | 5 | 4 |
|  | b) | Given X(z)=zz-0.7 find the Z-transform of yn=xn-3un-3. | 5 | 5 | 5 |
| OR |  |  |  |  |  |
| 11 | a) | Show how Z-transform is used to determine the system output using correlation in the Z-domain. | 5 | 5 | 3 |
|  | b) | Discuss the importance of ROC in determining system stability and causality in Z-domain. | 5 | 5 | 2 |

---oo0oo---

Page 2 of 2