SET-II

| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

II B.Tech I Semester Regular/Supplementary Examinations December-2025

A6AEO3 SOLID MECHANICS FOR AERONAUTICS

(Aeronautical)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 1M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Relate two equations used to find the forces in compound bars made of two materials subjected to tension. | 1 | 1 | 2 |
|  | b) | Define Stresses due to temperature. | 1 | 1 | 1 |
|  | c) | Define simple bending theory | 1 | 2 | 1 |
|  | d) | Define the term point of contra flexure. | 1 | 2 | 1 |
|  | e) | List out the methods to estimate the deflection of beams. | 1 | 3 | 2 |
|  | f) | Define Maxwell reciprocal theorem. | 1 | 3 | 1 |
|  | g) | Define shear stress. | 1 | 4 | 1 |
|  | h) | Write the torsional equation. | 1 | 4 | 1 |
|  | i) | What is Mohr’s circle? | 1 | 5 | 1 |
|  | j) | A thin cylinder closed at both ends is subjected to an internal pressure of 2MPa. Its internal diameter is 1m and the wall thickness is 10 mm. What is the maximum shear stress in the cylinder material? | 1 | 5 | 2 |

PART- B 			5 x 10M = 50Marks

| 2 | UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | M | CO | BL |
|  | a) | A steel rod 15 m long is at a temperature of 15°C. Find the free expansion of the length when the temperature is raised to 65°C. Find the temperature stress produced when: (i) The expansion of the rod is prevented; (ii) The rod is permitted to expand by 6 mm. Take: α = 12 × 10–6 per°C, and E = 200 GN/m2. | 5 | 1 | 3 |
|  | b) | A rod is 2 m long at a temperature of 100C. Find the expansion of the rod, when the temperature is raised to 800C. If the expansion is prevented, Find the stress induced in the material of the rod, Take E=1.0 x105 MN/m2 and α = 0.000012 per degree centigrade. | 5 | 1 | 3 |
| OR |  |  |  |  |  |
| 3 | a) | Two brass road and one steel rod as shown supports a load. If the stress in steel and brass not to exceed 120N/mm2 and 60N/mm2, find the safe load that can be supported. Es = 2*105 N/mm2 and Eb = 105 N/mm2. The cross-sectional area of steel is 1500mm2 and each brass rod is 1000 mm2. | 5 | 1 | 3 |
|  | b) | A load of 2MN is applied on a short concrete column 500 mm x 500 mm. The column is reinforced with four steel bars of 10 mm diameter, one in each corner. Find the stresses in the concrete and steel bars. Take E for steel as 2.1 x 105 N/mm2 and for concrete as 1.4 x 104 N/mm2. | 5 | 1 | 3 |
| UNIT-II |  |  |  |  |  |
| 4 | a) | A wooden beam of 100mm wide and 150mm deep is simply supported over a span of 4m. If the shear force at the section of beam is 4500N, find the shear stress at a distance of 25mm above the neutral axis. | 5 | 1 | 2 |
|  | b) | A 250 mm (depth) × 150 mm (width) rectangular beam is subjected to maximum bending moment of 750 kNm. Determine: (i) The maximum stress in the beam. (ii) If the value of E for the beam material is 200 GN/m2, find out the radius of curvature for that portion of the beam where the bending is maximum. | 5 | 1 | 3 |
| OR |  |  |  |  |  |
| 5 | a) | Draw shear force and bending moment diagrams for a SSB subjected to uniformly distributed load of w/unit length over entire span of length L. | 5 | 1 | 2 |
|  | b) | An aluminium plate of 150 mm width and 25 mm thickness is bend into a circular arc of radius 8m. Determine the maximum bending stress and the moment produced by the maximum bending stress. Take E= 1x 105 N/mm2. | 5 | 1 | 3 |
| UNIT-III |  |  |  |  |  |
| 6 |  | A SSB of 8m span carries a point load of 10 KN at its centre. It also subjects to a UDL of 1KN/m over its entire span. Find the maximum deflection of beam. Give E =200 KN/mm2, I= 200 x 106 mm4. | 10 | 1 | 3 |
| OR |  |  |  |  |  |
| 7 | a) | A load ‘W’ is placed at a distance ‘x’ from the fixed end of a cantilever of span ‘L’. Show that the deflection at the free end. | 5 | 1 | 2 |
|  | b) | For the beam shown below find the slope, deflection at ‘C’ and deflection at ‘B’. Take E = 2x105 N/mm2 and I = 108 mm4. | 5 | 1 | 2 |
| UNIT-IV |  |  |  |  |  |
| 8 |  | A steel shaft ABCD having a total length of 2.4 m consists of three lengths having different sections as follows: AB is hollow outside and inside diameter of 80 mm and 50 mm respectively and BC and CD are solid, BC having a diameter of 80 mm and CD a diameter of 70 mm. If the angle of twist is the same for each section, determine the length of each section and the total angle of twist if the maximum shear stress in the hollow portion in 50 N/mm2. Take C=8.2 x 104 N/mm2. | 10 | 1 | 2 |
| OR |  |  |  |  |  |
| 9 | a) | A hollow shaft of external and internal diameters as 100 mm and 40 mm is transmitted power at 120 rpm. Find the power the shaft can transmit, if the shearing stress is not exceeding 50 MPa. | 5 | 1 | 3 |
|  | b) | A hollow shaft is to transmit 300 KW at 80 rpm. If the shear stress is not to exceed 50 MN/m2 and diameter ratio is 3/7. Find the external and internal diameter if the twist is 1.20 and length is 2m. Assuming maximum torque is 20 % greater than mean. Take C= 80 GN/m | 5 | 1 | 3 |
| UNIT-V |  |  |  |  |  |
| 10 | a) | In a bi-axial stress problem, the stresses in x and y directions are σx = 200 MPa and σy =100 MPa. Find the maximum principal stress in MPa. | 5 | 1 | 3 |
|  | b) | A thin cylindrical shell has the following dimensions: Length = 3 m Inside diameter = 1 m Thickness of metal = 10 mm Internal pressure = 1.5 MPa Calculate the change in dimensions of the shell and the maximum intensity of shear stress induced. Take E = 200 GPa and Poisson’s ratio ν = 0.3. | 5 | 1 | 3 |
| OR |  |  |  |  |  |
| 11 | a) | A spherical Shell of internal diameter 0.9 m and of thickness 10 mm is subjected to an internal pressure of 1.4 N/mm2. Determine the increase in diameter and increase in volume. Take E=2 x 105 N/mm2 and Poisson ratio = 0.33. | 5 | 1 | 3 |
|  | b) | An element in a stressed material has tensile stress of 500 MN/m2 and a compressive stress of 350 MN/m2 acting on two mutually perpendicular planes and equal shear stresses of 100 MN/m2 on these planes. Find principal stresses and position of the principal planes. Find also maximum shearing stress. | 5 | 1 | 3 |

---oo0oo---

CO: Course Outcomes    BL Blooms Taxonomy Levels

Note:1. Font style : Bookman Old Style-Size-12
          2. Marks for sub question are to be mentioned in the question paper. The questions prepared                   
              shall be either (5M + 5M) or (10M) only. Other combinations are not acceptable.

3.	There shall not be morethan three 10M questions in the question paper

4. The Course Outcome (CO) and Bloom’s Taxonomy Level (BL) shall be mentioned for each question.