| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MLR20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	A5ME06

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

II B.Tech I Semester Supplementary Examinations December-2025

STRENGTH OF MATERIALS

(MECH)

Time: 3 Hours.								Max. Marks: 70

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 20 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 2M = 20Marks

| 1 | a) | Define plasticity. | 2 | CO1 | L1 |
| --- | --- | --- | --- | --- | --- |
|  | b) | Define stress. | 2 | CO1 | L1 |
|  | c) | What is UDL? | 2 | CO2 | L1 |
|  | d) | Define bending moment. | 2 | CO2 | L1 |
|  | e) | What is section modulus? | 2 | CO3 | L1 |
|  | f) | Write shear stress formula. | 2 | CO3 | L1 |
|  | g) | Define Mohr’s circle. | 2 | CO4 | L2 |
|  | h) | Define radius of curvature. | 2 | CO4 | L2 |
|  | i) | State formula for volumetric strain in thin cylinders. | 2 | CO5 | L2 |
|  | j) | Define radial stress in thick cylinder. | 2 | CO5 | L1 |

PART- B 			5 x 10M = 50Marks

| 2 |  | A bar of varying section has lengths: L₁ = 300 mm (area = 500 mm²) L₂ = 400 mm (area = 800 mm²) L₃ = 500 mm (area = 1200 mm²) Axial load = 70 kN Find total extension. (E = 2x105 N/mm2) | 10 | CO1 | L3 |
| --- | --- | --- | --- | --- | --- |
| OR |  |  |  |  |  |
| 3 | a) | A steel rod of length 1 m is strained by 0.0008. If Poisson’s ratio = 0.28, determine lateral strain and change in volume. | 5 | CO1 | L3 |
|  | b) | A bar absorbs 2 Joules of strain energy when stretched by 1 mm under axial pull. Determine the value of load if cross-sectional area = 300 mm², length = 800 mm. | 5 | CO1 | L3 |
| Page 1 of 2 |  |  |  |  |  |
| 4 | a) | Draw SFD & BMD for a cantilever of length 4 m carrying UDL 2 kN/m. | 5 | CO2 | L3 |
|  | b) | For a simply supported beam of span 5 m with point load 12 kN at midspan, compute support reactions and bending moment at midspan. | 5 | CO2 | L3 |
| OR |  |  |  |  |  |
| 5 | a) | A cantilever beam of 6 m carries UDL 4 kN/m for first 3 m and point load 10 kN at 5 m. Draw SFD. | 5 | CO2 | L3 |
|  | b) | A simply supported beam of span 8 m carries an UDL of 12kN/m for 6 m from the left support. Draw draw BMD and locate the point of contraflexure. | 5 | CO2 | L4 |
| 6 |  | A T-section having flange 200 mm × 20 mm and web 150 mm × 20 mm is subjected to bending moment 50 kN·m. Determine maximum tensile and compressive stresses. | 10 | CO3 | L3 |
| OR |  |  |  |  |  |
| 7 | a) | A beam of I-section has: Flange = 200×20 mm, Web = 300×15 mm Shear force = 60 kN Find max shear stress. | 5 | CO3 | L3 |
|  | b) | A circular beam of diameter 150 mm is subjected to bending moment 20 kN·m. Find max bending stress. | 5 | CO3 | L3 |
| 8 |  | At a point in a beam, the stresses are: σx = 80 MPa (tensile), σy = –20 MPa (compressive), τxy = 30 MPa Find principal stresses using analytical method. | 10 | CO4 | L4 |
| OR |  |  |  |  |  |
| 9 | a) | A simply supported beam of span 4 m carries point load 18 kN at center. Find deflection at center. Take E = 200 GPa and I = 4×10⁶ mm⁴ | 5 | CO4 | L3 |
|  | b) | A cantilever of length 2 m carries UDL 3 kN/m. Find slope at free end. Take E = 210 GPa and I = 2.5×10⁶ mm⁴ | 5 | CO4 | L3 |
| 10 |  | A thin spherical shell of diameter 1 m and thickness 8 mm is subjected to internal pressure 1.8 MPa. Determine hoop stress and change in volume for same sphere. Take E = 2x105 N/mm2, and µ=0.3 | 10 | CO5 | L3 |
| OR |  |  |  |  |  |
| 11 | a) | A thick cylinder has internal radius 80 mm, external radius 180 mm, and internal pressure 25 MPa. Determine radial stress at inner and outer surfaces. | 5 | CO5 | L4 |
|  | b) | A thick cylinder has internal radius 100 mm, external radius 160 mm and internal pressure 30 MPa. Using Lame’s equations, Compute hoop stress at mid-thickness. | 5 | CO5 | L3 |

---oo0oo---

Page 2 of 2