| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MLR20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: A5ME09

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

II B.Tech I Semester Supplementary Examinations December-2025

THERMODYNAMICS

(MECH)

Time: 3 Hours.								Max. Marks: 70

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 20 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 2M = 20Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | What is meant by thermodynamics system? | 2 | 1 | 1 |
|  | b) | Define intensive properties | 2 | 1 | 1 |
|  | c) | State Carnot’s Theorem. | 2 | 2 | 1 |
|  | d) | State the Kelvin –Planck statement of second law of Thermodynamics. | 2 | 2 | 1 |
|  | e) | Define the sensible heat of water. | 2 | 3 | 1 |
|  | f) | Define dryness fraction. | 2 | 3 | 1 |
|  | g) | Distinguish between ideal and real gas | 2 | 4 | 2 |
|  | h) | What is compressibility factor? | 2 | 4 | 1 |
|  | i) | Define dry bulb temperature | 2 | 5 | 1 |
|  | j) | Define the humidification process | 2 | 5 | 1 |

PART- B 			5 x 10M = 50Marks

| 2 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Explain the following? i)Specific Heat ii) Thermodynamic Cycle | 5 | 1 | 2 |
|  | b) | Explain the First Law of Thermodynamics. Formulate the equation for heat in non-flow reversible constant pressure process. | 5 | 1 | 2 |
| OR |  |  |  |  |  |
| 3 | a) | List the difference between a closed system and an open system | 5 | 1 | 1 |
|  | b) | Compare the cyclic process and non-cyclic process. | 5 | 1 | 4 |
| Page 1 of 2 |  |  |  |  |  |
| 4 | a) | State the concept of entropy of gas and availability and unavailability | 5 | 2 | 2 |
|  | b) | The air in a system expands from a temperature of 600C to 3000C at a constant pressure of 2 bars. Calculate the heat transfer, work done and change in intemal energy. The mass of the air is 0.6 Kg. Assume Cp:1.02 KJ/KgK and Cv:0.71 KJ/KgK for air. | 5 | 2 | 2 |
| OR |  |  |  |  |  |
| 5 | a) | Explain reversible and irreversible process. | 5 | 2 | 2 |
|  | b) | What is difference between a heat pump and refrigerator? | 5 | 2 | 2 |
| 6 |  | Explain in detail about Throttling Calorimeter with neat sketch. | 10 | 3 | 2 |
| OR |  |  |  |  |  |
| 7 | a) | State phase rule of pure substances. | 5 | 3 | 2 |
|  | b) | Differentiate between saturated liquid and compressed liquid. | 5 | 3 | 2 |
| 8 |  | Derive the Clausius- Clapreyon equation. | 10 | 4 | 4 |
| OR |  |  |  |  |  |
| 9 | a) | Derive the ratio of heat capacities. | 5 | 4 | 2 |
|  | b) | A gas mixture consists of 7 kg nitrogen and 2 kg oxygen, at 4 bar and 27˚C. Calculate the mole fraction, partial pressure, molar mass, gas constant, volume and density. | 5 | 4 | 2 |
| 10 |  | The sling psychrometer in a laboratory test recorded the following readings: Dry bulb temperature = 35°C Wet bulb temperature = 25°C. Calculate the following: (i) Specific humidity (ii) Relative humidity (iii) Vapour density in air (iv) Dew point temperature Take atmospheric pressure = 1.0132 bar | 10 | 5 | 2 |
| OR |  |  |  |  |  |
| 11 | a) | Describe any two psychometric processes with a neat sketch | 5 | 5 | 2 |
|  | b) | Differentiate between Dry bulb temperature and Wet bulb temperature | 5 | 5 | 2 |

---oo0oo---

Page 2 of 2