| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	A6EE60

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

II B.Tech I Semester Regular/Supplementary Examinations December-2025

BASIC ELECTRICAL AND ELECTRONICS ENGINEERING

(AERO)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			           10 x 1M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | A 100Ω resistance is directly switched on across a 10 V battery. What is the current through resistor? | 1 | 1 | 2 |
|  | b) | Write down the expression of equivalent resistance for ‘n’- number of resistors in parallel connection? | 1 | 1 | 1 |
|  | c) | What are the losses in a DC machine? | 1 | 2 | 1 |
|  | d) | State the Working Principle of DC Generator? | 1 | 2 | 1 |
|  | e) | What are the various parts of Transformer? | 1 | 3 | 1 |
|  | f) | Define the slip? | 1 | 3 | 1 |
|  | g) | What do you mean by forward biasing of a p-n junction? | 1 | 4 | 1 |
|  | h) | What is a Zener diode? Draw its symbol? | 1 | 4 | 1 |
|  | i) | What is NPN transistor? | 1 | 5 | 1 |
|  | j) | Define the saturation region of transistor? | 1 | 5 | 1 |

PART- B 			            5 x 10M = 50Marks

| 2 | a) | Explain in detail about Kirchhoff’s Current and Voltage Law? | 5 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | b) | Find the Equivalent resistance of the circuit? | 5 | 1 | 2 |
| OR |  |  |  |  |  |
| 3 | a) | Define and explain the different types of elements in brief? | 5 | 1 | 2 |
|  | b) | Derive the expression for Delta connected resistances in terms of Star connected resistances? | 5 | 1 | 3 |
| Page 1 of 2 |  |  |  |  |  |
| 4 | a) | Derive the expression for E.M.F equation of DC Generator? | 5 | 2 | 3 |
|  | b) | The wave connected armature of a two- pole 300 V generator has 500 conductors and runs at 400 rpm. Calculate the flux per pole? | 5 | 2 | 3 |
| OR |  |  |  |  |  |
| 5 |  | Derive an expression for the torque developed in the armature of a DC motor? | 10 | 2 | 3 |
| 6 |  | Write the operating principle of 3 phase Induction motor and explain the Torque-Slip characteristics of a 3-phase Induction motor? | 10 | 3 | 2 |
| OR |  |  |  |  |  |
| 7 | a) | Explain the construction and working principle of 1-ph transformer? | 5 | 3 | 2 |
|  | b) | 4-pole, 3-phase induction motor operates from a supply whose frequency is 50Hz. Calculate : (i). the speed at which the magnetic field of the stator is rotating. (ii). the speed of the rotor when the slip is 0.04. (iii). the frequency of the rotor currents when the slip is 0.03. (iv). the frequency of the rotor currents at standstill. | 5 | 3 | 3 |
| 8 |  | With neat diagram, explain the working principle of Bridge Rectifier. Draw its input and Output waveforms? | 10 | 4 | 2 |
| OR |  |  |  |  |  |
| 9 | a) | Explain the V-I Characteristics of a PN junction diode when it is forward and reverse biased? | 5 | 4 | 2 |
|  | b) | Explain about i). Avalanche effect ii). Zener effect. | 5 | 4 | 3 |
| 10 |  | With a neat sketch Explain the input and output and current gain characteristics of a transistor in common base (CB) configuration? | 10 | 5 | 2 |
| OR |  |  |  |  |  |
| 11 | a) | With the neat sketch explain the operation of PNP transistor? | 5 | 5 | 2 |
|  | b) | Describe the working of a transistor amplifier with neat diagrams? | 5 | 5 | 2 |

---oo0oo---

Page 2 of 2