| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	A6EC07

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

II B.Tech I Semester Regular/Supplementary Examinations December-2025

Electronic Measurements and Instrumentation

(ECE)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			       10 x 1M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Define “steady-state error.” | 1 | 1 | 1 |
|  | b) | Mention any two functions of a multimeter. | 1 | 1 | 1 |
|  | c) | Mention one application of a capacitance meter. | 1 | 2 | 1 |
|  | d) | What type of display is used in digital pH meters? | 1 | 2 | 1 |
|  | e) | What is a sweep voltage? | 1 | 3 | 1 |
|  | f) | What is the function of the channel selector? | 1 | 3 | 1 |
|  | g) | How is the quality factor (Q) of a coil described? | 1 | 4 | 1 |
|  | h) | Which parameter does a field strength meter indicate in dB? | 1 | 4 | 1 |
|  | i) | What is meant by transducer sensitivity? | 1 | 5 | 1 |
|  | j) | Define “loading effect” in transducers. | 1 | 5 | 1 |

PART- B 			        5 x 10M = 50Marks

| 2 | a) | Explain the different static characteristics of measuring instruments. | 5 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | b) | ### Explain accuracy vs precision with examples. | 5 | 1 | 3 |
| OR |  |  |  |  |  |
| 3 | a) | Discuss the different measurement quantities and how they are classified. | 5 | 1 | 2 |
|  | b) | Explain the principle, types, and applications of moving-iron (MI) instruments. | 5 | 1 | 4 |
| 4 | a) | Outline any single technique used in measuring capacitance digitally. | 5 | 2 | 5 |
|  | b) | Explain the principle and operation of a digital pH meter. | 5 | 2 | 3 |
| Page 1 of 2 |  |  |  |  |  |
| OR |  |  |  |  |  |
| 5 | a) | How does an RF signal generator operate based on its principle? | 5 | 2 | 4 |
|  | b) | Discuss how a frequency synthesizer operates. | 5 | 2 | 2 |
| 6 |  | Explain how Lissajous patterns are used for measuring frequency and phase difference. | 10 | 3 | 6 |
| OR |  |  |  |  |  |
| 7 | a) | Explain the operation of a triggered sweep CRO. | 5 | 3 | 4 |
|  | b) | Compare dual-beam and dual-trace CROs. | 5 | 3 | 3 |
| 8 |  | Explain in detail the construction, working principle, and applications of a Q-meter. Derive the expression for Q-factor and discuss the various sources of error in Q-measurements | 10 | 4 | 5 |
| OR |  |  |  |  |  |
| 9 | a) | Describe the Schering bridge with a neat diagram and explain how capacitance and dissipation factor are measured | 5 | 4 | 4 |
|  | b) | Write a short note on transistor testers. How are NPN and PNP transistors identified? | 5 | 4 | 3 |
| 10 | a) | How are transducers categorized into active and passive types? Provide examples. | 5 | 5 | 3 |
|  | b) | Explain how transducers are interfaced to measuring systems. | 5 | 5 | 2 |
| OR |  |  |  |  |  |
| 11 | a) | Explain the working and applications of a data logger. | 5 | 5 | 3 |
|  | b) | Describe the basic elements of a digital data acquisition system with a neat block diagram. | 5 | 5 | 4 |

---oo0oo---

Page 2 of 2