| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	B52128

MLR INSTITUTE OF TECHNOLOGY

(Autonomous)

II M.Tech I Sem Regular/Supplementary Examination January-2026

REFRIGERATION AND HVAC

(TE)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part- A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part – B consists 5 units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 1M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | What is meant by superheating in a vapour compression system? | 1 | 1 | 1 |
|  | b) | Write about the various refrigeration cycles? | 1 | 1 | 1 |
|  | c) | Write the normal boiling point of hydrogen | 1 | 2 | 1 |
|  | d) | What is meant by “COP”. | 1 | 2 | 1 |
|  | e) | What is a vortex tube? | 1 | 3 | 1 |
|  | f) | What is Bootstrap air refrigeration? | 1 | 3 | 1 |
|  | g) | Define psychometric. | 1 | 4 | 1 |
|  | h) | Differentiate between dry bulb temperature and wet bulb temperature. | 1 | 4 | 2 |
|  | i) | Name any two common types of refrigents? | 1 | 5 | 1 |
|  | j) | What is the function of a Chiller in an air-conditioning plant? | 1 | 5 | 1 |

PART- B 			5 x 10M = 50Marks

| 2 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  |  | With the help of a neat p-h or T-s diagram, explain the ideal vapour compression refrigeration cycle? | 10 | 1 | 3 |
| OR |  |  |  |  |  |
| 3 |  | A Vapour compression refrigerator works between the pressure limits of 60 bar and 25 bar. The working fluid is just dry at the end of compression and there is no under cooling of the liquid before the expansion valve. Determine (i). C.O.P of the cycle (ii). Capacity of the refrigerator if the fluid flow is at the rate of 5 kg/min. | 10 | 1 | 3 |
| Page 1 of 2 |  |  |  |  |  |
| 4 |  | Discuss process of production process dry ices with neat diagram. | 10 | 2 | 4 |
| OR |  |  |  |  |  |
| 5 | a) | Compare vapour compression and aqua–ammonia vapour absorption systems | 5 | 2 | 4 |
|  | b) | State advantages of the three fluid system used in domestic absorption refrigerators | 5 | 2 | 3 |
| 6 |  | With neat diagrams, discuss the Bell column refrigeration system. | 10 | 3 | 4 |
| OR |  |  |  |  |  |
| 7 |  | Draw a schematic of a Pulse Tube Refrigeration System and explain the functions of its components, Why pulse tube system achieve cryogenic temperatures? | 10 | 3 | 4 |
| 8 | a) | Discuss the requirements of comfort air-conditioning for human? | 5 | 4 | 3 |
|  | b) | Explain the construction of a psychometric chart. Discuss about its coordinate axes and curves. | 5 | 4 | 2 |
| OR |  |  |  |  |  |
| 9 |  | Explain the working of principle of thermo electric refrigeration system with neat diagram. | 10 | 4 | 3 |
| 10 |  | Explain, with the help of a psychometric chart, the concepts of RSHF, ESHF and GSHF for a multi-zone air-conditioning system | 10 | 5 | 4 |
| OR |  |  |  |  |  |
| 11 |  | Describe the construction and applications of axial flow? Write about the centrifugal fans used in air-conditioning systems | 10 | 5 | 3 |

---oo0oo---

Page 2 of 2