| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: A6ME23

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

III B.Tech I Semester Regular/Supplementary Examinations Novemeber-2025

MACHINE DESIGN

(MECH)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 1M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Define Bearing characteristic number. | 1 | 1 | 1 |
|  | b) | What is a rolling contact bearing? | 1 | 1 | 1 |
|  | c) | Provide general proportions of connecting rod cross section in terms of shell thickness. | 1 | 2 | 1 |
|  | d) | List the various materials used for the piston. | 1 | 2 | 1 |
|  | e) | State two applications of power screws. | 1 | 3 | 2 |
|  | f) | What is meant by helix angle? | 1 | 3 | 1 |
|  | g) | What is a belt drive? | 1 | 4 | 1 |
|  | h) | Name the materials commonly used for belt manufacture | 1 | 4 | 1 |
|  | i) | Define gear ratio. | 1 | 5 | 1 |
|  | j) | Name two types of gears used for power transmission | 1 | 5 | 1 |

PART- B 			5 x 10M = 50Marks

| 2 |  | M | CO | BL |
| --- | --- | --- | --- | --- |
|  | Design a journal bearing for a centrifugal pump from the following data : Load on the journal = 20000 N; Speed of the journal = 900 r.p.m.; Type of oil is SAE 10, for which the absolute viscosity at 55°C = 0.017 kg/m-s; Ambient temperature of oil = 15.5°C ; Maximum bearing pressure for the pump = 1.5 N / mm2. Calculate also mass of the lubricating oil required for artificial cooling, if rise of temperature of oil be limited to 10°C. Heat dissipation coefficient = 1232 W/m2/°C. | 10 | 1 | 4 |
| OR |  |  |  |  |
| 3 | Select a single row deep groove ball bearing for a radial load of 4000 N and an axial load of 5000 N, operating at a speed of 1600 r.p.m. for an average life of 5 years at 10 hours per day. Assume uniform and steady load. | 10 | 1 | 4 |
| Page 1 of 2 |  |  |  |  |
| 4 | Design a cast iron piston (find the dimensions of piston head and piston rings) for a single acting four stroke engine for the following data: Cylinder bore = 100 mm ; Stroke = 125 mm ; Maximum gas pressure = 5 N/mm2 ; Indicated mean effective pressure = 0.75N/mm2 ; Mechanical efficiency = 80% ; Fuel consumption = 0.15 kg per brake power per hour ; Higher calorific value of fuel = 42 × 103 kJ/kg ; Speed = 2000 r.p.m. Any other data required for the design may be assumed. | 10 | 2 | 4 |
| OR |  |  |  |  |
| 5 | Write the design procedure of piston. | 10 | 2 | 3 |
| 6 | A single-threaded power screw, with ISO metric square threads is used to raise a load of 300 kN. The nominal diameter is 100 mm and the pitch is 12 mm. The coefficient of friction at the screw threads is 0.15. Neglecting collar friction, calculate (i) torque required to raise the load; (ii) torque required to lower the load; and (iii) efficiency of the screw. | 10 | 3 | 3 |
| OR |  |  |  |  |
| 7 | A screw jack is to lift a load of 80 kN through a height of 400 mm. The elastic strength of screw material in tension and compression is 200 MPa and in shear 120 MPa. The material for nut is phosphor-bronze for which the elastic limit may be taken as 100 MPa in tension, 90 MPa in compression and 80 MPa in shear. The bearing pressure between the nut and the screw is not to exceed 18 N/mm2. Design and draw the screw jack. The design should include the design of a. screw, b. nut | 10 | 3 | 3 |
| 8 | Leather belt 9 mm × 250 mm is used to drive a cast iron pulley 900 mm in diameter at 336 r.p.m. If the active arc on the smaller pulley is 120° and the stress in tight side is 2 MPa, find the power capacity of the belt. The density of leather is 980 kg/m3, and the coefficient of friction of leather on cast iron is 0.35 | 10 | 4 | 3 |
| OR |  |  |  |  |
| 9 | Select a wire rope for a vertical mine hoist to lift a load of 55 kN from a depth 300 metres. A rope speed of 500 metres / min is to be attained in 10 seconds. | 10 | 4 | 4 |
| 10 | A bronze spur pinion rotating at 600 r.p.m. drives a cast iron spur gear at a transmission ratio of 4:1. The allowable static stresses for the bronze pinion and cast iron gear are 84 MPa and 105 MPa respectively. The pinion has 16 standard 20° full depth involute teeth of module 8 mm. The face width of both the gears is 90 mm. Find the power that can be transmitted from the standpoint of strength. | 10 | 5 | 3 |
| OR |  |  |  |  |
| 11 | Helical cast steel gear with 30° helix angle has to transmit 35 kW at 1500 r.p.m. If the gear has 24 teeth, determine the necessary module, pitch diameter and face width for 20° full depth teeth. The static stress for cast steel may be taken as 56 MPa. The width of face may be taken as 3 times the normal pitch. What would be the end thrust on the gear? The tooth factor for 20° full depth involute gear may be taken as 0.154- 0.912TE where TE represents the equivalent number of teeth. | 10 | 5 | 3 |

Page 2 of 2