| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	A6EC21

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

III B.Tech I Semester Regular/Supplementary Examinations November-2025

MICROPROCESSORS AND MICROCONTROLLERS

(ECE)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 1M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | List any four features of 8085 microprocessor. | 1 | 1 | 1 |
|  | b) | What is the function of the Bus Interface Unit (BIU)? | 1 | 1 | 1 |
|  | c) | List any four assembler directives in 8086. | 1 | 2 | 1 |
|  | d) | Give an example of immediate addressing mode. | 1 | 2 | 1 |
|  | e) | What is the function of the 8255 PPI? | 1 | 3 | 1 |
|  | f) | What is the function of 8251 USART? | 1 | 3 | 1 |
|  | g) | Mention any two differences between a microprocessor and a microcontroller. | 1 | 4 | 1 |
|  | h) | Mention the function of the ALU in 8051. | 1 | 4 | 1 |
|  | i) | How many timers are available in 8051? | 1 | 5 | 1 |
|  | j) | What is the vector address of external interrupt 0? | 1 | 5 | 1 |

PART- B 			5 x 10M = 50Marks

| 2 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Explain the basic architecture of 8086 microprocessor with a neat block diagram and describe the function of each block. | 5 | 1 | 2 |
|  | b) | Explain the concept of memory segmentation in 8086 microprocessor with a neat diagram. | 5 | 1 | 2 |
| OR |  |  |  |  |  |
| 3 | a) | A segment register contains 2000H and the offset address is 1000H. Calculate the physical address and explain the steps involved. | 5 | 1 | 3 |
|  | b) | Explain the register organization of 8086 microprocessor with a neat diagram. | 5 | 1 | 2 |
| Page 1 of 2 |  |  |  |  |  |
| 4 | a) | Explain in detail the different addressing modes of 8086 with examples. | 5 | 2 | 2 |
|  | b) | Classify and explain the instruction set of 8086 based on the type of operations performed. | 5 | 2 | 4 |
| OR |  |  |  |  |  |
| 5 | a) | Write an assembly program to compare two strings using the CMPS instruction and display whether they are equal. | 5 | 2 | 3 |
|  | b) | Write an 8086 assembly program to sort an array of numbers in ascending order. | 5 | 2 | 3 |
| 6 |  | Explain the architecture and pin diagram of the 8255 Programmable Peripheral Interface. | 10 | 3 | 2 |
| OR |  |  |  |  |  |
| 7 | a) | Explain the architecture and pin configuration of 8251 USART with a neat diagram. | 5 | 3 | 2 |
|  | b) | Discuss the DMA transfer modes supported by 8257 and explain their operations. | 5 | 3 | 3 |
| 8 |  | Explain the architecture of 8051 microcontroller with a neat block diagram and describe the function of each block. | 10 | 4 | 2 |
| OR |  |  |  |  |  |
| 9 | a) | Discuss external memory organization in 8051, including code and data memory. | 5 | 4 | 3 |
|  | b) | Explain the role of EA, PSEN, and ALE signals in memory access. | 5 | 4 | 2 |
| 10 | a) | Explain different modes of 8051 timers and draw register formats for TMOD and TCON registers. | 5 | 5 | 2 |
|  | b) | Write an assemble language program to 8051 micro controller to blink alternate LED’s of port 1 using timer 1 mode 1. | 5 | 5 | 3 |
| OR |  |  |  |  |  |
| 11 |  | Explain the architecture and key features of ARM processor. | 10 | 5 | 2 |

---oo0oo---

Page 2 of 2