| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	A6EE15

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

III B.Tech I Semester Regular/Supplementary Examinations Novemeber-2025

POWER ELECTRONICS

(EEE)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 1M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | What is Sunbber circuit? | 1 | 1 | BL1 |
|  | b) | List the different types of commutation circuits used in SCR? | 1 | 1 | BL1 |
|  | c) | Define the average load voltage? | 1 | 2 | BL1 |
|  | d) | What are the advantages of three-phase converters? | 1 | 2 | BL1 |
|  | e) | List the applications of cycloconverter? | 1 | 3 | BL1 |
|  | f) | What are the types of AC voltage controllers? | 1 | 3 | BL1 |
|  | g) | Define the duty cycle? | 1 | 4 | BL1 |
|  | h) | List the advantages of Boost converter? | 1 | 4 | BL1 |
|  | i) | What are the types of inverters? | 1 | 5 | BL1 |
|  | j) | Define the conduction angle? | 1 | 5 | BL1 |

PART- B 			5 x 10M = 50Marks

| 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | M | CO | BL |
|  | a) | Describe the construction and working of SCR with its characteristics? | 5 | 1 | BL2 |
|  | b) | What are the different turning – ON methods of a thyristor? Explain any one method in detail? | 5 | 1 | BL3 |
| OR |  |  |  |  |  |
| 3 | a) | For an SCR, the gate- cathode characteristics has straight line slope of 130. For trigger source voltage of 15 V, and allowable gate power dissipation of 0.5 watts, calculate the value of gate source resistance? | 5 | 1 | BL5 |
|  | b) | Discuss the working and characteristics of a GTO? | 5 | 1 | BL2 |
| 4 | a) | Discuss the operation of a single-phase full converter with freewheeling diode? | 5 | 2 | BL2 |
|  | b) | Describe the working of three phase dual converter in circulating mode? | 5 | 2 | BL2 |
| OR |  |  |  |  |  |
| 5 | a) | Discuss the effect of source inductance on converter performance? | 5 | 2 | BL3 |
|  | b) | A single phase fully controlled bridge converter is connected to R-L load with R = 10 Ω and L = 6 mH. The converter is supplied from 230 V, 50 Hz ac supply. Determine average and rms load current. | 5 | 2 | BL5 |
| 6 |  | Briefly explain the operation of TRIAC in different modes? | 10 | 3 | BL4 |
| OR |  |  |  |  |  |
| 7 | a) | Explain the operation of two SCRs in anti-parallel configuration? | 5 | 3 | BL3 |
|  | b) | Describe the working of bridge-type cycloconverter? | 5 | 3 | BL2 |
| 8 |  | Explain the different control strategies in DC-DC circuits and list the applications of DC–DC converters? | 10 | 4 | BL4 |
| OR |  |  |  |  |  |
| 9 | a) | Discuss the principle of operation of Buck-Boost converter? | 5 | 4 | BL2 |
|  | b) | Explain the working of boost converter with relevant waveform? | 5 | 4 | BL2 |
| 10 | a) | Explain the operation of a single-phase full-bridge inverter? | 5 | 5 | BL2 |
|  | b) | Explain the working of capacitor commutated type current source inverter? | 5 | 5 | BL2 |
| OR |  |  |  |  |  |
| 11 |  | Describe the PWM techniques used in inverters? | 10 | 5 | BL4 |

---oo0oo---