| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: A6EE16

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

III B.Tech I Semester Regular/Supplementary Examinations Novemeber-2025

POWER SYSTEMS -II

(EEE)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 1M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | What is dielectric stress? | 1 | 1 | 1 |
|  | b) | Define inter-sheath grading. | 1 | 1 | 1 |
|  | c) | What is a feeder? | 1 | 2 | 1 |
|  | d) | Compare overhead and underground distribution systems | 1 | 2 | 1 |
|  | e) | What is the function of a Peterson coil? | 1 | 3 | 1 |
|  | f) | What is meant by counterpoise in grounding systems? | 1 | 3 | 1 |
|  | g) | What is the per-unit value of any base quantity? | 1 | 4 | 1 |
|  | h) | Define symmetrical components. | 1 | 4 | 1 |
|  | i) | What are sequence networks? | 1 | 5 | 1 |
|  | j) | What is a symmetrical fault? | 1 | 5 | 1 |

PART- B 			5 x 10M = 50Marks

| 2 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Derive the expression for the capacitance of a 1-core cable. | 5 | 1 | 3 |
|  | b) | Explain the electric stress distribution in a single-core cable with a neat diagram. | 5 | 1 | 2 |
| OR |  |  |  |  |  |
| 3 |  | A cable has conductor radius r1=1 cm, inner sheath at r2=3 cm and outer sheath at r3=6 cm. The conductor is at 110 kV relative to the outer sheath (outer sheath earthed). For inter-sheath grading that makes stresses roughly equal in the two insulation regions, what should the potential of the inner sheath (relative to outer sheath) be? | 10 | 1 | 3 |
| Page 1 of 2 |  |  |  |  |  |
| 4 | a) | Derive the expression for voltage drop in a DC distributor fed at both ends with unequal voltages. | 5 | 2 | 3 |
|  | b) | A 500 m long distributor is fed at A (250 V) and B (240 V). It supplies: 60 A at 100 m, 40 A at 300 m, 30 A at 400 m. Resistance = 0.15 Ω/km. Find the minimum potential point and current supplied from each end. | 5 | 2 | 3 |
| OR |  |  |  |  |  |
| 5 | a) | Explain voltage drop calculations in AC distributors considering power factor at each load point. | 5 | 2 | 2 |
|  | b) | Differentiate between radial, ring, and interconnected distribution systems. | 5 | 2 | 3 |
| 6 |  | Explain about valve type and expulsion type lightning arresters. | 10 | 3 | 2 |
| OR |  |  |  |  |  |
| 7 | a) | Explain in detail overvoltage due to arcing ground and describe how it can be eliminated using a Peterson coil. | 5 | 3 | 2 |
|  | b) | What is insulation coordination? Explain its importance and describe the use of volt-time characteristics in coordination. | 5 | 3 | 2 |
| 8 |  | A 3-phase unbalanced voltage system has voltages VR,VY,VB Derive expressions for V0,V1,V2(zero, positive, and negative sequence voltages). | 10 | 4 | 3 |
| OR |  |  |  |  |  |
| 9 | a) | Explain the concept of positive, negative, and zero sequence components with phasor diagrams. | 5 | 4 | 2 |
|  | b) | Discuss the principle of symmetrical components. Derive the necessary equations to convert Symmetrical components into phase quantities. | 5 | 4 | 3 |
| 10 | a) | Explain the procedure for calculating fault current using symmetrical components. | 5 | 5 | 2 |
|  | b) | Derive the expression for current and voltage during a single line-to-ground (LG) fault without fault impedance. | 5 | 5 | 3 |
| OR |  |  |  |  |  |
| 11 | a) | Compare the sequence network connections for LG, LL, and LLG faults in a tabular form. | 5 | 5 | 4 |
|  | b) | Derive the general equation for fault current in an LLG fault with fault impedance Zf | 5 | 5 | 3 |

---oo0oo---

Page 2 of 2