| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: A6EC19

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

III B.Tech I Semester Regular/Supplementary Examinations Novemeber-2025

LINEAR AND DIGITAL INTEGRATED CIRCUIT APPLICATIONS

(ECE)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 1M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Give any two reasons why an open loop op-amp is not suitable for linear applications. | 1 | 1 | 2 |
|  | b) | Mention any four ideal op-amp characteristics. | 1 | 1 | 2 |
|  | c) | What is virtual ground concept? Explain. | 1 | 2 | 2 |
|  | d) | What is a comparator? Mention its applications. | 1 | 2 | 2 |
|  | e) | Mention the advantages of active filters over passive filters. | 1 | 3 | 2 |
|  | f) | Determine the resistor value, RA of a monostable Multivibrator using IC 555 timer to generate a pulse width of 1msec. (Assume capacitance value, C=0.01µF). | 1 | 3 | 3 |
|  | g) | Write the features of IC 723. | 1 | 4 | 2 |
|  | h) | Define the conversion time of an A/D converter. | 1 | 4 | 2 |
|  | i) | What is race around condition? How it is avoided? | 1 | 5 | 4 |
|  | j) | Draw the circuit of Ring Counter. | 1 | 5 | 4 |

PART- B 			5 x 10M = 50Marks

| 2 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Explain the characteristics of an ideal op-amp. | 5 | 1 | 2 |
|  | b) | For open loop difference amplifier with 741C and with gain of 20, 0000, sketch input and output when (i) 1mV at Non inverting terminal and 2mV at inverting terminal (ii) 1mV at non inverting terminal and zero volt at inverting terminal and (iii) zero volt at non inverting terminal and 2mV at inverting terminal. Assume all the input signals sinusoidal with 1kHz. | 5 | 1 | 4 |
| OR Page 1 of 2 |  |  |  |  |  |
| 3 | a) | Discuss the three basic types of linear IC packages. Explain the characteristics of each. | 5 | 1 | 2 |
|  | b) | Outline the function of various blocks in block diagram of an Op-amp with neat sketch. | 5 | 1 | 4 |
| 4 |  | What is an instrumentation amplifier? Illustrate the working of three op-amp Instrumentation amplifier. Give its applications. | 10 | 2 | 4 |
| OR |  |  |  |  |  |
| 5 | a) | Draw and explain the operation of practical differentiator. | 5 | 2 | 4 |
|  | b) | Explain the operation of Schmitt Trigger using Op-Amp and also derive the expression for its hysteresis voltage. | 5 | 2 | 2 |
| 6 | a) | Design a second order Butterworth low-pass filter having higher cut-off frequency of 1kHz. | 5 | 3 | 3 |
|  | b) | With a neat circuit diagram, explain the operation of IC 555 timer in astable mode and also derive the expression for % duty cycle. | 5 | 3 | 4 |
| OR |  |  |  |  |  |
| 7 | a) | What is an All pass filter? With a neat diagram, explain the working of all pass filter. | 5 | 3 | 2 |
|  | b) | Draw the functional block diagram of IC555 and explain its operation in detail. | 5 | 3 | 2 |
| 8 | a) | What is a voltage regulator? List different types of voltage regulators and state its advantages | 5 | 4 | 2 |
|  | b) | Give the schematic circuit diagram of the fastest A/D converter and explain its operation. | 5 | 4 | 2 |
| OR |  |  |  |  |  |
| 9 | a) | With a neat circuit diagram, explain about switching regulator. | 5 | 4 | 2 |
|  | b) | Draw the circuit of weighted resistor DAC and derive expression for output analog voltage VO. | 5 | 4 | 2 |
| 10 | a) | Design and 8 to 3 decoder, explain its operation with a truth table. | 5 | 5 | 4 |
|  | b) | Draw and explain 4-bit universal shift register. | 5 | 5 | 2 |
| OR |  |  |  |  |  |
| 11 | a) | Explain about 74 X 148 priority encoder. | 5 | 5 | 4 |
|  | b) | Draw the logic diagram of 74×163 binary counter and explain its operation. | 5 | 5 | 2 |

---oo0oo---

Page 2 of 2