| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: A6ME27

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

III B.Tech I Semester Regular/Supplementary Examinations Novemeber-2025

THERMAL ENGINEERIN-II

(MECH)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 1M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | What is mean temperature of heat addition? | 1 | 1 | 1 |
|  | b) | Define boiler. | 1 | 1 | 1 |
|  | c) | What is nozzle efficiency? | 1 | 2 | 1 |
|  | d) | Name the types of condensers. | 1 | 2 | 1 |
|  | e) | What is compounding of steam turbines? | 1 | 3 | 1 |
|  | f) | Define blade efficiency. | 1 | 3 | 1 |
|  | g) | Mention any two merits of closed gas turbine cycles. | 1 | 4 | 1 |
|  | h) | What is intercooling in gas turbines? | 1 | 4 | 1 |
|  | i) | Define air conditioning. | 1 | 5 | 1 |
|  | j) | Mention any two uses of P–h chart. | 1 | 5 | 1 |

PART- B 			5 x 10M = 50Marks

| 2 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Explain the working of Loeffler boiler with neat sketch. | 5 | 1 | 2 |
|  | b) | Explain the working of Lamont boiler with neat sketch. | 5 | 1 | 2 |
| OR |  |  |  |  |  |
| 3 |  | A regenerative Rankine cycle operates between 40 bar and 0.1 bar. One open feed water heater is used, and steam is bled at 5 bar. Steam enters the turbine at 400 °C. Find: (i) Mass fraction of bled steam (ii) Network output per kg of steam (iii) Thermal efficiency. | 10 | 1 | 4 |
| Page 1 of 2 |  |  |  |  |  |
| 4 |  | A convergent–divergent nozzle expands steam from 10 bar and 300°C to 1.5 bar. If the mass flow is 1.2 kg/s and nozzle efficiency 88%, find throat area, exit area, and exit velocity. | 10 | 2 | 3 |
| OR |  |  |  |  |  |
| 5 | a) | Explain the classification and applications of steam condensers | 5 | 2 | 2 |
|  | b) | Compare jet condenser and surface condenser. | 5 | 2 | 2 |
| 6 |  | An impulse turbine runs at 3000 rpm with nozzle angle of 20°, blade angle at inlet 30°, steam velocity 700 m/s, and blade velocity coefficient of 0.85. Find power developed, blade velocity, and diagram efficiency for 1.5 kg/s steam flow. | 10 | 3 | 3 |
| OR |  |  |  |  |  |
| 7 |  | Derive the maximum efficiency of a reaction turbine. | 10 | 3 | 4 |
| 8 |  | A gas turbine unit has a pressure ratio of 6:1. Air enters the compressor at 1 bar and 20 °C. The maximum cycle temperature is 800 °C. Assuming isentropic efficiency of compressor = 0.8 and turbine = 0.85, calculate: (i) Compressor work (ii) Turbine work (iii) Network output (iv) Thermal efficiency.. | 10 | 4 | 3 |
| OR |  |  |  |  |  |
| 9 | a) | Explain with neat sketch closed cycle gas turbine and draw its T-S diagram and also derive the expression for its thermal efficiency, And also mention advantages and disadvantages. | 10 | 4 | 4 |
| 10 | a) | Explain Vapour absorption refrigeration system with schematic. | 5 | 5 | 2 |
|  | b) | Differentiate between vapour compression refrigeration (VCR) and vapour absorption refrigeration (VAR) systems. | 5 | 5 | 2 |
| OR |  |  |  |  |  |
| 11 | a) | Explain the classification and working principle of any one air conditioning systems. | 5 | 5 | 2 |
|  | b) | Write short notes on requirements of typical A/C plant. | 5 | 5 | 2 |

---oo0oo---

Page 2 of 2