| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: A6ME24

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

III B.Tech I Semester Regular/Supplementary Examinations Novemeber-2025

THEORY OF MACHINES-II

(MECH)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 1M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | What is gyroscope and write formula for its magnitude. | 1 | 1 | 1 |
|  | b) | Define ‘inertia force’ and ‘inertia torque | 1 | 1 | 2 |
|  | c) | Explain the following: (i) Limiting friction (ii) Angle of friction | 1 | 2 | 2 |
|  | d) | Distinguish between brakes and dynamometers. | 1 | 2 | 4 |
|  | e) | What is the function of a flywheel? | 1 | 3 | 1 |
|  | f) | How is the governor differ from that of a flywheel ? | 1 | 3 | 4 |
|  | g) | Why is balancing of rotating parts necessary for high speed engines? | 1 | 4 | 4 |
|  | h) | Define primary and secondary balancing. | 1 | 4 | 2 |
|  | i) | Define free vibrations and forced vibrations | 1 | 5 | 1 |
|  | j) | What is the main cause of torsional vibration in shafts? | 1 | 5 | 1 |

PART- B 			5 x 10M = 50Marks

| 2 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Explain the gyroscopic effect on aeroplanes. | 5 | 1 | 2 |
|  | b) | An aero plane makes a complete half circle of 50 m radius, towards left, when flying at 200 km per hr. The rotary engine and the propeller of the plane have a mass of 400 kg and a radius of gyration of 0.3 m. The engine rotates at 2400 r.p.m. clockwise when viewed from the rear. Find the gyroscopic couple on the aircraft and state its effect on it. | 5 | 1 | 3 |
| OR |  |  |  |  |  |
| 3 |  | The crank and connecting rod of a steam engine are 0.3 m and 1.5 m in length. The crank rotates at 180 r.p.m. clockwise. Determine the velocity and acceleration of the piston when the crank is at 40 degrees from the inner dead centre position. Also determine the position of the crank for zero acceleration of the piston. | 10 | 1 | 3 |
| Page 1 of 3 |  |  |  |  |  |
| 4 | a) | A conical pivot supports a load of 20 kN, the cone angle is 120º and the intensity of normal pressure is not to exceed 0.3 N/mm2. The external diameter is twice the internal diameter. Find the outer and inner radii of the bearing surface. If the shaft rotates at 200 r.p.m. and the coefficient of friction is 0.1, find the power absorbed in friction. Assume uniform pressure. | 5 | 2 | 3 |
|  | b) | Describe with a neat sketch the working of a single plate friction clutch | 5 | 2 | 2 |
| OR |  |  |  |  |  |
| 5 | a) | A bicycle and rider of mass 100 kg are travelling at the rate of 16 km/h on a level road. A brake is applied to the rear wheel which is 0.9 m in diameter, and this is the only resistance acting. How far will the bicycle travel and how many turns will it make before it comes to rest? The pressure applied on the brake is 100 N and µ = 0.05. | 5 | 2 | 3 |
|  | b) | Describe the construction and operation of a Prony brake or rope brake absorption dynamometer. | 5 | 2 | 2 |
| 6 | a) | The mass of flywheel of an engine is 6.5 tonnes, and the radius of gyration is 1.8 metres. It is found from the turning moment diagram that the fluctuation of energy is 56 kN-m. If the mean speed of the engine is 120 r.p.m., find the maximum and minimum speeds. | 5 | 3 | 3 |
|  | b) | The flywheel of a steam engine has a radius of gyration of 1 m and mass 2500 kg. The starting torque of the steam engine is 1500 N-m and may be assumed constant. Determine: 1. the angular acceleration of the flywheel, and 2. the kinetic energy of the flywheel after 10 seconds from the start. | 5 | 3 | 3 |
| OR |  |  |  |  |  |
| 7 | The arms of a Porter governor are each 250 mm long and pivoted on the governor axis. The mass of each ball is 5 kg and the mass of the central sleeve is 30 kg. The radius of rotation of the balls is 150 mm when the sleeve begins to rise and reaches a value of 200 mm for maximum speed. Determine the speed range of the governor. If the friction at the sleeve is equivalent of 20 N of load at the sleeve, determine how the speed range is modified. |  | 10 | 3 | 4 |
| 8 | a) | Explain clearly the terms ‘static balancing’ and ‘dynamic balancing’. State the necessary conditions to achieve them. | 5 | 4 | 2 |
|  | b) | Four masses m1, m2, m3 and m4 are 200 kg, 300 kg, 240 kg and 260 kg respectively. The corresponding radii of rotation are 0.2 m, 0.15 m, 0.25 m and 0.3 m respectively and the angles between successive masses are 45°, 75° and 135°. Find the position and magnitude of the balance mass required, if its radius of rotation is 0.2 m. | 5 | 4 | 3 |
| OR Page 2 of 3 |  |  |  |  |  |
| 9 | a) | A single cylinder reciprocating engine has speed 240 r.p.m., stroke 300 mm, mass of reciprocating parts 50 kg, mass of revolving parts at 150 mm radius 37 kg. If two third of the reciprocating parts and all the revolving parts are to be balanced, find: 1. The balance mass required at a radius of 400 mm, and 2. The residual unbalanced force when the crank has rotated 60° from top dead centre. | 5 | 4 | 3 |
|  | b) | Define the following terms for an uncoupled two-cylinder locomotive engine: (a) Variation is tractive force; (b) Swaying couple; and (c) Hammer blow. | 5 | 4 | 2 |
| 10 | a) | What are the causes and effects of vibrations? | 5 | 5 | 2 |
|  | b) | Calculate the whirling speed of a shaft 20 mm diameter and 0.6 m long carrying a mass of 1 kg at its mid-point. The density of the shaft material is 40 Mg/m3, and Young’s modulus is 200 GN/m2. Assume the shaft to be freely supported | 5 | 5 | 3 |
| OR |  |  |  |  |  |
| 11 | a) | Derive the expression for the frequency of free torsional vibrations for shaft fixed at one end and carries load on the free end | 5 | 5 | 3 |
|  | b) | A shaft of 100 mm diameter and 1 metre long has one of its ends fixed and the other end carries a disc of mass 500 kg at a radius of gyration of 450 mm. The modulus of rigidity for the shaft material is 80 GN/m2. Determine the frequency of torsional vibrations. | 5 | 5 | 3 |

---oo0oo---

Page 3 of 3