| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: A6EE31

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

III B.Tech I Semester Regular/Supplementary Examinations Novemeber-2025

WIND AND SOLAR ENERGY SYSTEMS

(EEE)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 1M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Define tip speed ratio (TSR). | 1 | 1 | 1 |
|  | b) | What is pitch control in wind turbines? | 1 | 1 | 2 |
|  | c) | Name the generator that uses permanent magnets for excitation. | 1 | 2 | 2 |
|  | d) | What is the function of converter control in a wind energy conversion system | 1 | 2 | 2 |
|  | e) | Define zenith angle | 1 | 3 | 1 |
|  | f) | Define the term “solar day length. | 1 | 3 | 1 |
|  | g) | What is the typical efficiency of mono crystalline silicon cells? | 1 | 4 | 2 |
|  | h) | What parameter is controlled in converter control to achieve MPPT? | 1 | 4 | 2 |
|  | i) | Define fault ride-through (FRT) capability. | 1 | 5 | 1 |
|  | j) | What are the main causes of voltage flicker in wind farms? | 1 | 5 | 3 |

PART- B 			5 x 10M = 50Marks

| 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | M | CO | BL |
|  | a) | Explain the relationship between wind speed and power output of a wind turbine | 5 | 1 | 2 |
|  | b) | Explain the physics of wind and how wind energy is generated | 5 | 1 | 2 |
| OR |  |  |  |  |  |
| 3 | a) | Write short notes on: (a) Tip speed ratio (b) Stall and pitch control (c) Wind speed distribution. | 5 | 1 | 1 |
|  | b) | Explain Betz limit and efficiency of a wind turbine. | 5 | 1 | 2 |
| Page 1 of 2 |  |  |  |  |  |
| 4 | a) | Discuss the challenges and benefits of integrating modern wind turbines into the power grid. | 5 | 2 | 3 |
|  | b) | Compare the performance and applications of Induction Generators and PMSGs. | 5 | 2 | 4 |
| OR |  |  |  |  |  |
| 5 | a) | Explain the construction and working of a Permanent Magnet Synchronous Generator (PMSG) | 5 | 2 | 2 |
|  | b) | Write short notes on power electronic converters used in wind turbines. | 5 | 2 | 1 |
| 6 |  | Explain in detail the solar radiation geometry. Derive the expressions for declination angle, hour angle, and altitude angle. Illustrate how these vary with time and location. | 10 | 3 | 2 |
| OR |  |  |  |  |  |
| 7 | a) | Explain the concept of Earth–Sun angles with a neat diagram. | 5 | 3 | 2 |
|  | b) | Describe how to estimate monthly average daily solar radiation using empirical relations. | 5 | 3 | 1 |
| 8 |  | Explain the principle and operation of Maximum Power Point Tracking (MPPT) in PV systems and MPPT algorithms. | 10 | 4 | 2 |
| OR |  |  |  |  |  |
| 9 | a) | Compare the characteristics of monocrystalline, polycrystalline. | 5 | 4 | 4 |
|  | b) | Describe the basic operation of a boost converter used in solar PV systems. | 5 | 4 | 1 |
| 10 | a) | Explain the importance of grid code technical requirements for renewable energy systems. | 5 | 5 | 2 |
|  | b) | Write a short note on power quality issues related to grid-connected solar PV systems. | 5 | 5 | 1 |
| OR |  |  |  |  |  |
| 11 | a) | Discuss the challenges in power system interconnection of large-scale wind and solar plants. | 5 | 5 | 4 |
|  | b) | Explain the role of power electronic converters in maintaining grid stability. | 5 | 5 | 2 |

---oo0oo---

Page 2 of 2