| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	A6EE46

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

IV B.Tech I Semester Regular Examinations Novemeber-2025

HIGH VOLTAGE ENGINEERING

(EEE)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 1M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Name the different types of electrical discharges that occur in gases. | 1 | 1 | 1 |
|  | b) | Define corona discharge. | 1 | 1 | 1 |
|  | c) | Which circuit is commonly used for generating high-voltage DC? | 1 | 2 | 1 |
|  | d) | State the main function of a triggering gap in impulse voltage generators. | 1 | 2 | 1 |
|  | e) | Define dielectric loss factor. | 1 | 3 | 1 |
|  | f) | Why is a resistive divider unsuitable for very fast impulse voltage measurements? | 1 | 3 | 1 |
|  | g) | Name any two devices used for protection against overvoltages. | 1 | 4 | 1 |
|  | h) | State what is meant by a stepped leader in a lightning discharge. | 1 | 4 | 1 |
|  | i) | What is the purpose of standardization in high-voltage testing? | 1 | 5 | 1 |
|  | j) | Define type test and routine test as per IEC standards. | 1 | 5 | 1 |

PART- B 			5 x 10M = 50Marks

| 2 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Discuss the drawbacks of the Townsend’s mechanism? | 5 | 1 | 2 |
|  | b) | What are the factors that influence the conduction in pure liquid dielectrics and in commercial liquid dielectrics? | 5 | 1 | 1 |
| OR |  |  |  |  |  |
| 3 | a) | Explain briefly various theories of breakdown in liquid dielectrics. | 5 | 1 | 2 |
|  | b) | Apply the concept of the streamer mechanism to explain how a breakdown occurs in a non-uniform electric field. Illustrate with the help of a example. | 5 | 1 | 3 |
| Page 1 of 2 |  |  |  |  |  |
| 4 | a) | Explain how a multistage impulse generator is charged and discharged to produce standard lightning impulses. Support your answer with a circuit layout. | 5 | 2 | 3 |
|  | b) | List the various methods used for generating high voltages in laboratories and state one application for each. | 5 | 2 | 2 |
| OR |  |  |  |  |  |
| 5 | a) | With neat sketches, explain the working of a Cockcroft–Walton voltage multiplier circuit used for high DC generation. | 5 | 2 | 2 |
|  | b) | Describe with neat diagrams the working of a voltage doubler circuit used for high-voltage DC generation. | 5 | 2 | 2 |
| 6 | a) | Describe the construction and operation of a sphere gap for measuring peak voltages. How does air density affect its accuracy? | 5 | 3 | 3 |
|  | b) | List the different methods used for measuring peak voltages in high-voltage systems. Mention their applications. | 5 | 3 | 2 |
| OR |  |  |  |  |  |
| 7 | a) | Describe the role of a cathode ray oscillograph (CRO) in impulse voltage measurement. Explain how it captures the waveform. | 5 | 3 | 2 |
|  | b) | Describe the electrical detection method of partial discharge measurement and sketch a typical test circuit. | 5 | 3 | 3 |
| 8 | a) | Explain how the stepped leader initiates a lightning flash and the mechanism of return stroke formation. | 5 | 4 | 2 |
|  | b) | Explain the difference between lightning surges and switching surges with suitable waveforms. | 5 | 4 | 2 |
| OR |  |  |  |  |  |
| 9 | a) | Describe the process of switching overvoltages in high-voltage systems and the factors influencing their magnitude. | 5 | 4 | 3 |
|  | b) | Describe the working principle of surge modifier and state its advantages. | 5 | 4 | 3 |
| 10 | a) | Describe the role of standard waveforms and reference conditions specified in IEC standards for HV testing. | 5 | 5 | 3 |
|  | b) | Explain the power frequency withstand test and its significance for high-voltage insulators. | 5 | 5 | 2 |
| OR |  |  |  |  |  |
| 11 | a) | Explain the induced overvoltage test and its purpose in transformer testing. | 5 | 5 | 2 |
|  | b) | Discuss the requirements and safety measures to be followed in high-voltage testing facilities for HV Labs. | 5 | 5 | 3 |

---oo0oo---

Page 2 of 2