| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	A6EC32

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

IV B.Tech I Semester Regular Examinations Novemeber-2025

MICROWAVE ENGINEERING

(ECE)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 1M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | What is the dominant mode in a rectangular waveguide? | 1 | 1 | 1 |
|  | b) | Define characteristic impedance (Z₀). | 1 | 1 | 1 |
|  | c) | Define a waveguide discontinuity. | 1 | 2 | 2 |
|  | d) | Name two types of waveguide attenuators. | 1 | 2 | 1 |
|  | e) | What is velocity modulation in an O-type tube? | 1 | 3 | 2 |
|  | f) | State the importance of slow-wave structures in TWTs. | 1 | 3 | 1 |
|  | g) | What is meant by cross-field effect? | 1 | 4 | 2 |
|  | h) | State the Hull cut-off condition in magnetrons. | 1 | 4 | 1 |
|  | i) | What is the function of the attenuator in a microwave bench? | 1 | 5 | 1 |
|  | j) | Explain the function of a slotted line section. | 1 | 5 | 1 |

PART- B 			5 x 10M = 50Marks

| 2 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Explain the microwave spectrum and its various frequency bands, along with their corresponding applications. | 5 | 1 | 1 |
|  | b) | Derive the relation between phase velocity, group velocity, and wave velocity. | 5 | 1 | 3 |
| OR |  |  |  |  |  |
| 3 | a) | Derive the expression for the resonant frequency of a rectangular cavity. | 5 | 1 | 2 |
|  | b) | Explain different types of losses in microstrip lines and how to minimize them. | 5 | 1 | 1 |
| Page 1 of 2 |  |  |  |  |  |
| 4 | a) | Explain different coupling mechanisms used in waveguides — probe, loop, and aperture types. | 5 | 2 | 2 |
|  | b) | Discuss how attenuation is achieved and measured in waveguides. | 5 | 2 | 1 |
| OR |  |  |  |  |  |
| 5 | a) | Explain the principle of Faraday rotation in ferrite devices. | 5 | 2 | 1 |
|  | b) | Illustrate the working principle and major applications of a gyrator. | 5 | 2 | 2 |
| 6 | a) | Explain the structure and working of a 2-cavity Klystron. Discuss its velocity modulation and Applegate diagram. | 10 | 3 | 4 |
| OR |  |  |  |  |  |
| 7 | a) | What are the common limitations of conventional microwave tubes at microwave frequencies? How do these limitations affect their performance? | 5 | 3 | 3 |
|  | b) | What are slow-wave structures in TWTs? Discuss their importance in determining the performance and efficiency of the Traveling Wave Tube. | 5 | 3 | 2 |
| 8 |  | Derive the Hull cut-off condition for magnetron operation. | 10 | 4 | 1 |
| OR |  |  |  |  |  |
| 9 | a) | What is the π-mode operation in a magnetron? Why is it preferred? | 5 | 4 | 4 |
|  | b) | Describe the working principle of a Gunn diode with the help of RWH theory. | 5 | 4 | 2 |
| 10 |  | Explain the different components of a microwave bench setup with a neat block diagram. | 10 | 5 | 2 |
| OR |  |  |  |  |  |
| 11 | a) | Mention the importance of the slotted line section in microwave measurement techniques. | 5 | 5 | 5 |
|  | b) | Describe the procedure for attenuation measurement using a variable attenuator. | 5 | 5 | 4 |

---oo0oo---

Page 2 of 2