| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: A6EE26

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

IV B.Tech I Semester Regular Examinations Novemeber-2025

POWER SYSTEM OPERATION AND CONTROL

(EEE)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 1M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Define Load Flow Study and mention its main purpose | 1 | 1 | 1 |
|  | b) | State the advantages of the Newton–Raphson method over the Gauss–Seidel method. | 1 | 1 | 2 |
|  | c) | Define Heat Rate Curve and mention its significance in economic operation. | 1 | 2 | 1 |
|  | d) | Define Incremental Fuel Cost (IFC) of a generating unit. | 1 | 2 | 1 |
|  | e) | Define steady-state stability and transient stability of a power system. | 1 | 3 | 1 |
|  | f) | State the significance of the power–angle curve in stability analysis. | 1 | 3 | 2 |
|  | g) | State the main function of a speed governor in a power plant. | 1 | 4 | 1 |
|  | h) | What is meant by the excitation system time constant, and why is it important? | 1 | 4 | 2 |
|  | i) | Define Load Frequency Control (LFC) and explain its necessity in a power system. | 1 | 5 | 1 |
|  | j) | Define voltage control in a power system. | 1 | 5 | 1 |

PART- B 			5 x 10M = 50Marks

| 2 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Explain the classification of buses in a power system and specify known and unknown quantities for each bus type. | 5 | 1 | 2 |
|  | b) | Form the Y-bus matrix for a 3-bus system with line admittances: - Between Bus 1–2: j0.2 p.u. - Between Bus 2–3: j0.25 p.u. - Between Bus 1–3: j0.4 p.u. Use the inspection method. | 5 | 1 | 3 |
| OR Page 1 of 2 |  |  |  |  |  |
| 3 |  | Derive the load flow equation for a n-bus system | 10 | 1 | 3 |
| 4 | a) | Explain the significance of equality and inequality constraints in the economic allocation of generation among different plants in a system | 5 | 2 | 3 |
|  | b) | Two generating units have the following cost characteristics: C1=0.002P12+10P1+200 C2=0.003P22+8P2+180 Find the economic load allocation for a total load demand of 200 MW, neglecting transmission losses. | 5 | 2 | 3 |
| OR |  |  |  |  |  |
| 5 | a) | Derive the transmission line loss formula for a system consisting of n-generating plants supplying several loads inter connected through a transmission networks. State any assumptions are made. | 5 | 2 | 4 |
|  | b) | A generating unit has the following incremental fuel cost characteristic: dCdP=0.1P+25 Rs/MWh Find the total cost of generation when the unit supplies 200 MW of power. | 5 | 2 | 3 |
| 6 |  | Derive the swing equation for a synchronous machine connected to an infinite bus and explain how it is used to determine system stability. Also discuss the factors affecting transient stability and the methods to improve it. | 10 | 3 | 4 |
| OR |  |  |  |  |  |
| 7 | a) | Explain the difference between steady-state, dynamic, and transient stability with neat sketches. | 5 | 3 | 2 |
|  | b) | Explain the equal area criterion? | 5 | 3 | 3 |
| 8 |  | Derive the combined transfer function of the governor–turbine–generator system. Explain the physical significance of each block and describe how the system reacts to a sudden increase in load. | 10 | 4 | 4 |
| OR |  |  |  |  |  |
| 9 |  | Explain the block diagram representation of an excitation system and describe its control action in regulating terminal voltage. | 10 | 4 | 2 |
| 10 | a) | Explain the necessity of keeping frequency constant in a power system and list the main factors affecting frequency. | 5 | 5 | 2 |
|  | b) | Explain the steady-state response of a single-area LFC system and the effect of integral control on frequency deviation. | 5 | 5 | 4 |
| OR |  |  |  |  |  |
| 11 |  | Explain the generation and absorption of reactive power by synchronous machines, transmission lines, and loads. | 10 | 5 | 2 |

---oo0oo---

Page 2 of 2