| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	B62103

MLR INSTITUTE OF TECHNOLOGY

(Autonomous)

I M.Tech I Sem Regular Examination February-2026

ADVANCED FLUID MECHANICS

(TE)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part- A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part – B consists 5 units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			5 x 2M = 10Marks

| 1 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | a) | Define circulation and vorticity. Write the mathematical expression for vorticity in Cartesian coordinates. | 2 | 1 | 1 |
|  | b) | State the Navier–Stokes equations for a viscous compressible flow and explain the physical significance of each term. | 2 | 2 | 1 |
|  | c) | Define local drag coefficient and mean drag coefficient. State their significance in boundary layer analysis. | 2 | 3 | 1 |
|  | d) | Explain the Prandtl mixing length hypothesis and state its limitations. | 2 | 4 | 1 |
|  | e) | What are Mach angle and Mach cone? State their significance in supersonic flow | 2 | 5 | 1 |

PART- B 			5 x 10M = 50Marks

| 2 |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
|  | Derive the three-dimensional continuity equation in Cartesian coordinate system for a compressible flow. |  | 10 | 1 | 3 |
| OR |  |  |  |  |  |
| 3 | a) | Derive the condition for irrotational flow | 5 | 1 | 3 |
|  | b) | Establish the relationship between circulation and vorticity. | 5 | 1 | 2 |
| 4 | a) | Derive the velocity profile for Couette flow without pressure gradient | 5 | 2 | 3 |
|  | b) | Extend the analysis to Couette flow with pressure gradient and comment on the resulting velocity distribution | 5 | 2 | 2 |
| OR |  |  |  |  |  |
| 5 | Derive Navier-Stoke’s equation for viscous compressible flow |  | 10 | 2 | 3 |
| 6 | Explain the Blasius solution for laminar boundary layer flow over a flat plate. Discuss its significance and limitations. |  | 10 | 3 | 2 |
| OR Page 1 of 2 |  |  |  |  |  |
| 7 | Discuss boundary layer phenomena for flow over flat plate in detail. |  | 10 | 3 | 3 |
| 8 | a) | Discuss boundary layer control techniques used to delay separation and enhance lift on circular cylinders. | 5 | 4 | 2 |
|  | b) | Discuss the advantages and limitations of the k–ε model compared to simpler turbulence models. | 5 | 4 | 2 |
| OR |  |  |  |  |  |
| 9 | a) | Explain the phenomenon of Kármán vortex trail and its engineering significance | 5 | 4 | 2 |
|  | b) | Discuss boundary layer control techniques used to delay separation and enhance lift on circular cylinders. | 5 | 4 | 2 |
| 10 | Derive the property relationships in terms of Mach number for isentropic flow and explain their application to convergent–divergent nozzles |  | 10 | 5 | 3 |
| OR |  |  |  |  |  |
| 11 | a) | Derive the governing relations for a normal compressible shock. | 5 | 5 | 3 |
|  | b) | Explain oblique shock and Prandtl–Meyer expansion processes with suitable sketches and discuss their role in supersonic flow systems | 5 | 5 | 2 |

---oo0oo---

Page 2 of 2