| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	 B52119

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institution)

I M.TECH I Semester Regular/Supplementary Examinations, March-2024

ADVANCED IC ENGINES

(TE)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part- A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part – B consists 5 units. Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			        10 x 1M=10Marks

| 1. | a) | List the various assumptions made in Air Standard cycle Analysis | 1M | CO1 | BL1 |
| --- | --- | --- | --- | --- | --- |
|  | b) | Why SI engine requires a rich mixture during idling? | 1M | CO1 | BL1 |
|  | c) | Define the term Turbocharging. | 1M | CO2 | BL2 |
|  | d) | Write the importance of supercharging | 1M | CO2 | BL2 |
|  | e) | Describe briefly D-MPFI injection system. | 1M | CO3 | BL2 |
|  | f) | What are the major pollutants to be controlled? | 1M | CO3 | BL1 |
|  | g) | Describe the causes of hydrocarbon emissions from SI Engines. | 1M | CO4 | BL2 |
|  | h) | Write about the different types of alternative fuels available. | 1M | CO4 | BL2 |
|  | i) | List the advantages of HCCI engine. | 1M | CO5 | BL1 |
|  | j) | What are the factors to be consider for modifications in IC engines to suit bio-fuels? | 1M | CO5 | BL1 |

PART- B 			        5 x 10M=50Marks

| 2 | a) | Describe from the point of view of Fuel-air cycle analysis how does fuel-air ratio affect efficiency, maximum power, temperature and pressure in a cycle | 5M | CO1 | BL4 |
| --- | --- | --- | --- | --- | --- |
|  | b) | Calculate the percentage change in efficiency of a duel cycle having compression ratio = 16 and cutoff ratio of 10% of swept volume and if Cvincrease by 2% . Given T3/T2 = 1.67. | 5M | CO1 | BL3 |
| OR |  |  |  |  |  |
| 3 | a) | Explain the design and operating characteristics of SI and CI engines. | 5M | CO1 | BL2 |
|  | b) | Write about wankal engine with neat sketch. | 5M | CO1 | BL2 |
| 4 | a) | Describe the effect of supercharging on engine performance. | 5M | CO2 | BL2 |
|  | b) | Explain with a neat sketch the principle of exhaust turbocharging of single cylinder engine | 5M | CO2 | BL2 |
| OR Page 1 of 2 |  |  |  |  |  |
| 5 | a) | List what are the limitations of supercharging in an IC engine | 5M | CO2 | BL1 |
|  | b) | Explain turbocharging of a V type engine. | 5M | CO2 | BL2 |
| 6 | a) | Explain stages of combustion in SI Engine. | 5M | CO3 | BL2 |
|  | b) | Explain the stages of combustion in CI Engine with neat sketches. | 5M | CO3 | BL2 |
| OR |  |  |  |  |  |
| 7 | a) | Describe diesel fuel spray behaviour and spray structure with neat sketch. | 5M | CO3 | BL3 |
|  | b) | Illustrate the working of Common Rail Direct Injection system. | 5M | CO3 | BL3 |
| 8 | a) | Describe the thermal converter. How does it help to reduce emissions from engines. | 5M | CO4 | BL3 |
|  | b) | Explain how emissions be reduced using chemical methods. | 5M | CO4 | BL2 |
| OR |  |  |  |  |  |
| 9 | a) | Explain what is EGR and explain how it reduces the NOx formation. | 5M | CO4 | BL2 |
|  | b) | Explain about the swirl chamber. | 5M | CO4 | BL4 |
| 10 | a) | Tell a brief account of LPG being used as an alternate fuel in SI engine | 5M | CO5 | BL1 |
|  | b) | Explain the possibilities of using dual-fuel systems in engines. | 5M | CO5 | BL2 |
| OR |  |  |  |  |  |
| 11 | What is ignition delay? Mention the factors for reducing the ignition delay. |  | 10M | CO5 | BL1 |

---oo0oo---

Page  2 of 2