| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MLR20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	 B47601

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institution)

I M.Tech I Semester Supplementary Examinations, March-2024

AERODYNAMICS OF FLIGHT VEHICLES

(AEROSPACE)

Time: 3 Hours.								Max. Marks: 70

Note: 1. This question paper contains two parts A and B.

2. Part- A is Compulsory which carries 20 marks. Answer all Questions in part A.

3. Part – B consists 5 units. Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			        5 x 4M=20Marks

| 1. | a) | Define downwash and induced drag. | 4M | CO1 | BL1 |
| --- | --- | --- | --- | --- | --- |
|  | b) | Mention the design rules for wing-fuselage combination. | 4M | CO2 | BL2 |
|  | c) | Mention the similarity considerations for compressible boundary layers | 4M | CO3 | BL2 |
|  | d) | Draw the flow field around spheres and circular cylinders. | 4M | CO4 | BL2 |
|  | e) | Write short notes on Coanda effect. | 4M | CO5 | BL2 |

PART- B 			        5 x 10M=50Marks

| 2 |  | Derive the fundamental equation of thin aerofoil theory. | 10M | CO1 |  | BL2 |
| --- | --- | --- | --- | --- | --- | --- |
| OR |  |  |  |  |  |  |
| 3 | a) | Derive the relation for velocity induced at a point by an infinite straight vortex filament using Biot-Savart law. | 5M | CO1 |  | BL3 |
|  | b) | Explain the flow characteristics of symmetrical and unsymmetrical airfoils with a neat schematic of lift–coefficient variation with angle of attack. | 5M | CO1 |  | BL2 |
| 4 | a) | Show that for a perfect gas, the Mach number behind the normal shock (M2) is a function of only the Mach number ahead of the shock (M1). | 5M | CO2 |  | BL3 |
|  | b) | Air at 1 MPa and 300 K enters the Mach 2 Laval nozzle of a supersonic wind tunnel at a low velocity. If a normal shock wave is formed at the nozzle exit plane, determine the Mach number of the flow just behind the shock. | 5M | CO2 |  | BL3 |
| OR |  |  |  |  |  |  |
| 5 |  | Derive the Prandtl-Glauret compressibility correction relations for subsonic flows. | 10M | CO2 |  | BL3 |
| 6 |  | Derive the expressions for Rotation and Entropy Gradient in the Boundary Layer | 10M |  | CO3 | BL3 |
| OR |  |  |  |  |  |  |
| 7 | a) | Differentiate between laminar boundary layer in incompressible flow and compressible flow. | 5M | CO3 |  | BL2 |
|  | b) | Discuss the effects of pressure gradient and compressibility in boundary layer. | 5M | CO3 |  | BL2 |
| 8 |  | Compare laminar and turbulent mean velocity profiles for the same boundary layer thickness. | 10M |  | CO4 | BL2 |
| OR |  |  |  |  |  |  |
| 9 | a) | Discuss the methods to control laminar flow. | 5M | CO4 |  | BL2 |
|  | b) | List and discuss the various methods for experimentally detecting transition. | 5M | CO4 |  | BL2 |
| 10 | a) | List the various high-lift devices for aerofoils. | 5M | CO5 |  | BL2 |
|  | b) | What are secondary flows? Explain the formation of wing-fuselage trailing vortex. | 5M | CO5 |  | BL2 |
| OR |  |  |  |  |  |  |
| 11 | a) | Discuss the methods to control the circulation over the airfoil. | 5M | CO5 |  | BL2 |
|  | b) | Explain the flow field over delta wing. | 5M | CO5 |  | BL2 |

---oo0oo---