| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: B55834

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

II M.TECH I Semester Regular Examinations January/February-2024

OPTIMIZATION TECHNIQUES

(TE)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part- A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part – B consists 5 units. Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 				10 x 1 = 10Marks

| 1 | a) | Write about applications of operations research. | 1M | CO1 | BL1 |
| --- | --- | --- | --- | --- | --- |
|  | b) | What is the importance of Uni-Model function? | 1M | CO1 | BL1 |
|  | c) | Describe Kendall’s notation. | 1M | CO2 | BL1 |
|  | d) | Define Linear Programming problem. | 1M | CO2 | BL1 |
|  | e) | Explain the difference between the two-phase method and Big M method | 1M | CO3 | BL1 |
|  | f) | Explain about gaming theory. | 1M | CO3 | BL1 |
|  | g) | Define Inventory | 1M | CO4 | BL1 |
|  | h) | Define Geometric cutting plane algorithm | 1M | CO4 | BL1 |
|  | i) | Define polynomials | 1M | CO5 | BL1 |
|  | j) | Define Genetic algorithm | 1M | CO5 | BL2 |

PART- B				5 x 10 = 50Marks

| 2 | a) | Briefly explain the phases of operation research study. | 5M | CO1 | BL2 |
| --- | --- | --- | --- | --- | --- |
|  | b) | Explain briefly features of operation research approach. | 5M | CO1 | BL2 |
| OR |  |  |  |  |  |
| 3 |  | Solve the following LPP by graphical method Maximize Z = 5x 1 + 3x 2 Subject to constraints 2x 1 + x 2 ≤ 1000 x 1 ≤ 400 x 1 ≤ 700 x 1 , x 2 ≥ 0 | 10M | CO1 | BL3 |
| 4 |  | A company has three production facilities S1,S2 and S3 with production capacity of 7,9 and 18 units(in 100s) per week of the product respectively these units are to be shifted to four warehouses D1,D2,D3 and D4 with requirements of 5,6,7 and 14 units (in 100s) per week, respectively. D1; D2; D3; D4; Supply. S1; 19; 30; 50; 10; 7. S2; 70; 30; 40; 60; 9. S3; 40; 8; 17; 20; 18. Demand; 5; 8; 7; 14; 34. Use North-west corner method to find the feasible solution to the transportation problem. | 10M | CO2 | BL4 |
| OR |  |  |  |  |  |
| 5 |  | A department of a company has five employees with five jobs to be performed. The time (in hours) that each man takes to perform each job is given in effectiveness matrix Employees Jobs; I; II; III; IV; V. A; 10; 5; 13; 15; 16. B; 3; 9; 18; 13; 6. C; 10; 7; 2; 2; 2. D; 7; 11; 9; 7; 12. E; 7; 9; 10; 4; 12. How should the jobs allocate, one per employees so has to minimize the total man hours. | 10M | CO2 | BL4 |
| 6 |  | Describe a single server waiting line model give an example of real life for each of the following queueing models i. first come first served ii. Last come first served iii. Random pick service iv. Customers stay only if served instantly | 10M | CO3 | BL4 |
| OR |  |  |  |  |  |
| 7 | a) | Explain about Kuhn-Tucker condition to get optimal solution. | 5M | CO3 | BL2 |
|  | b) | Briefly explain optimization of single variable functions. | 5M | CO3 | BL2 |
| 8 | a) | Briefly explain optimization of multivariable functions. | 5M | CO4 | BL2 |
|  | b) | Explain factor effecting the inventory problem analysis | 5M | CO4 | BL2 |
| OR |  |  |  |  |  |
| 9 | a) | Explain about game theory concept along with managerial applications. | 5M | CO4 | BL2 |
|  | b) | Mention the importance Zero or One Algorithm? | 5M | CO4 | BL2 |
| 10 | a) | Explain briefly the following: (i) Random variables (ii) Probability density functions | 5M | CO5 | BL2 |
|  | b) | Explain the steps involved travelling salesman problem. | 5M | CO5 | BL2 |
| OR |  |  |  |  |  |
| 11 |  | Use dynamic program to solve the following problem. Minimize Z=y12+y22+y32 Subjected to the constraint y1+y2+y3=10 and y1,y2,y3>=0 | 10M | CO5 | BL4 |

---oo0oo---					Page 2 of 2