| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	B52128

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

II M.Tech I Semester Regular/Supplementary Examination January-2025

REFRIGERATION AND HVAC

(THERMAL ENGINEERING)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 		10 x 1M = 10Marks

| 1 | a) | Define undercooling | 1M | CO1 | BL1 |
| --- | --- | --- | --- | --- | --- |
|  | b) | List two advantages of vapour compression over air compression system | 1M | CO1 | BL1 |
|  | c) | Describe the role of pressure in the liquefaction of gases | 1M | CO2 | BL1 |
|  | d) | What is therefrigerant used in Vapour Absorption Systems | 1M | CO2 | BL1 |
|  | e) | Define an air refrigeration system | 1M | CO3 | BL1 |
|  | f) | What is the main principle behind the steam jet refrigeration system | 1M | CO3 | BL1 |
|  | g) | Define dew point temperature | 1M | CO4 | BL1 |
|  | h) | What does specific volume refer to in psychometrics. | 1M | CO4 | BL1 |
|  | i) | Describe the purpose of the condenser coil in an air conditioning system | 1M | CO5 | BL1 |
|  | j) | Give the difference between a window air conditioner and a split air conditioner | 1M | CO5 | BL1 |

PART- B 		5 x 10M = 50Marks

| 2 | a) | A refrigeration machine is required to produce ice at 0°C from water at 20 °C. Themachine has a condenser temperature of 298 K while the evaporator temperature is 268 K. The relativeefficiency of the machine is 50% and 6 kg of Freon-12 refrigerant is circulated through the system perminute. The refrigerant enters the compressor with a dryness fraction of 0. 6 Specific heat of water is 4.187 kJ/kg K and the latent heat of ice is 335 kJ/kg. Calculate the amount of ice produced in 24hours. The table of properties of Freon- 12 is given below Temp 0K; Liquid Heat KJ/Kg; Latent Heat KJ/Kg; Entropy of liquid KJ/Kg K. 298 268; 59.7 31.4; 138.0 154.0; 0.2232 0.1251. | 5M | CO1 |  | BL2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | b) | Explain the advantages of compound compression with intercooler over single stage compression. | 5M | CO1 |  | BL2 |
| OR Page 1 of 2 |  |  |  |  |  |  |
| 3 |  | Explain with neat sketches vapour compression refrigeration. | 10M | CO1 |  | BL2 |
| 4 |  | Explain bell column air refrigeration system with neat diagram | 10M | CO2 |  | BL2 |
| OR |  |  |  |  |  |  |
| 5 | a) | Discuss the principles and components of a liquefaction system. How does each component contribute to the process of converting a gas to a liquid | 5M | CO2 |  | BL1 |
|  | b) | Using the p-h (pressure-enthalpy) diagram, explain the process occurring in the Vapour Absorption Refrigeration System | 5M | CO2 |  | BL2 |
| 6 |  | Discuss a production of dry ice with neat diagram. | 10M |  | CO3 | BL2 |
| OR |  |  |  |  |  |  |
| 7 | a) | Discuss the role of steam generation in the overall energy efficiency of steam jet refrigeration systems | 5M | CO3 |  | BL1 |
|  | b) | Discuss the Advantages and disadvantages of Thermoelectric Refrigeration in Comparison to Conventional Cooling Methods. | 5M | CO3 |  | BL |
| 8 |  | Explain thermoelectric refrigeration working principle with neat diagram. | 10M |  | CO4 | BL2 |
| OR |  |  |  |  |  |  |
| 9 | a) | Describe few common mistakes done in calculating heating load. | 5M | CO4 |  | BL2 |
|  | b) | Explain briefly with a neat sketch a 'sling psychrometer | 5M | CO4 |  | BL2 |
| 10 | a) | Explain with a neat sketch an 'air-conditioning cycle | 5M | CO5 |  | BL2 |
|  | b) | Explain the difference between summer air-conditioning and winter air-conditioning | 5M | CO5 |  | BL2 |
| OR |  |  |  |  |  |  |
| 11 | a) | Explain the advantages of central system over unitary system of air-conditioning | 5M | CO5 |  | BL2 |
|  | b) | Advantages and disadvantages of the air refrigeration system. | 5M | CO5 |  | BL2 |

---oo0oo---

Page 2 of 2