| H.T NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MLR20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code:	 B47602

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institution)

I M.Tech I Semester Supplementary Examinations, August/September-2025

FLIGHT VEHICLE STRUCTURES

(ASE)

Time: 3 Hours.								Max. Marks: 70

Note: 1. This question paper contains two parts A and B.

2. Part- A is Compulsory which carries 20 marks. Answer all Questions in part A.

3. Part – B consists 5 units. Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			        5 x 4M=20Marks

| 1 | a) | List out the energy methods used in thin plate theory | 4M | CO1 | BL2 |
| --- | --- | --- | --- | --- | --- |
|  | b) | Give short note on inelastic buckling of plates | 4M | CO2 | BL1 |
|  | c) | List out the assumptions used for determining direct stress distribution of beams | 4M | CO3 | BL2 |
|  | d) | Explain Bredt–Batho theory | 4M | CO4 | BL1 |
|  | e) | Explain Structural Idealization with example | 4M | CO5 | BL2 |

PART- B 			        5 x 10M=50Marks

| 2 | a) | Briefly explain Thin Plates subjected to bending and twisting | 5M | CO1 | BL1 |
| --- | --- | --- | --- | --- | --- |
|  | b) | A plate 10mm thick is subjected to bending moments Mx equal to 10 Nm/mm and My equal to 5 Nm/mm. find the maximum twisting moment per unit length in the plate and the direction of the planes on which this occurs | 5M | CO1 | BL3 |
| OR |  |  |  |  |  |
| 3 |  | A thin rectangular plate a×b is simply supported along its edges and carries a uniformly distributed load of intensity q0. Determine the deflected form of the plate and the distribution of bending moment. | 10M | CO1 | BL3 |
| 4 | a) | Differentiate the complete diagonal tension and incomplete diagonal tension | 5M | CO2 | BL3 |
|  | b) | Explain buckling of rectangular thin plates | 5M | CO2 | BL2 |
| OR |  |  |  |  |  |
| 5 |  | Discuss about Failure stress in plates and stiffened panels | 10M | CO2 | BL3 |
| 6 |  | A thin-walled closed section beam has the singly symmetrical cross-section shown in Fig. Each wall of the section is flat and has the same thickness t and shear modulus G. Calculate the distance of the shear centre from point 4. | 10M | CO3 | BL4 |
| OR |  |  |  |  |  |
| 7 | a) | A beam having the cross-section shown in Fig. is subjected to a bending moment of 1200Nm in a vertical plane. Calculate the maximum direct stress due to bending. Ixx=1.09x106 mm4 , Iyy=1.31x106 mm4 , Ixy=0.34x106 mm4 | 5M | CO3 | BL3 |
|  | b) | Explain the Temperature effects on stress strain relations of beams | 5M | CO3 | BL2 |
| 8 |  | Find the angle of twist per unit length in the wing whose cross-section is shown in Fig. when it is subjected to a torque of 10 kN m. Find also the maximum shear stress in the section. G=25 000 N/mm2. Wall 12 (outer)=900 mm. Nose cell area=20 000mm2. | 10M | CO4 | BL3 |
| OR |  |  |  |  |  |
| 9 | a) | A thin-walled circular section beam has a diameter of 200mm and is 2m long; it is firmly restrained against rotation at each end. A concentrated torque of 30 kNm is applied to the beam at its mid-span point. If the maximum shear stress in the beam is limited to 200N/mm2 and the maximum angle of twist to 2◦, calculate the minimum thickness of the beam walls. Take G=25 000N/mm2. | 5M | CO4 | BL3 |
|  | b) | Briefly Explain Shear stress distribution of Combined open and closed section beams | 5M | CO4 | BL1 |
| 10 |  | Explain Shear flow Distribution of tapered Wing spar | 10M | CO5 | BL2 |
| OR |  |  |  |  |  |
| 11 |  | The central cell of a wing has the idealized section shown in Fig. If the lift and drag loads on the wing produce bending moments of −120000Nm and −30000Nm, respectively at the section shown, calculate the direct stresses in the booms. Neglect axial constraint effects and assume that the lift and drag vectors are in vertical and horizontal planes. Boom areas: B1 = B4 = B5 = B8 = 1000mm2 B2 = B3 = B6 = B7 = 600mm2 | 10M | CO5 | BL4 |

---oo0oo---

Page 3 of 3