| H.T NO. |  |  | R | 2 |  |  |  |  |  |  |  |  |  |  | MLR20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: B47612

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

I M.Tech II Semester Supplementary Examinations August/September-2025

ADVANCED FINITE ELEMENT METHODS

(ASE)

Time: 3 Hours.								Max. Marks: 70

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 20 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 2M = 20Marks

| 1 | a) | Explain the principle of minimum potential energy | 4M | BL2 | CO1 |
| --- | --- | --- | --- | --- | --- |
|  | b) | Draw the mode shapes of hermite shape functions | 4M | BL2 | CO2 |
|  | c) | Differentiate CST and LST Elements | 4M | BL2 | CO3 |
|  | d) | Explain iso-parametric representation of 4-noded quadrilateral element | 4M | BL2 | CO4 |
|  | e) | Explain the principle of Virtual work | 4M | BL2 | CO5 |

PART- B 			5 x 10M = 50Marks

| 2 | a) | Describe the properties of Global Stiffness Matrix (K) | 5M | BL2 | CO1 |
| --- | --- | --- | --- | --- | --- |
|  | b) | Derive the Strain Displacement matrix [B] for 1-D bar element | 5M | BL3 | CO1 |
| OR |  |  |  |  |  |
| 3 | a) | Explain Galerkin approach with suitable example | 5M | BL2 | CO1 |
|  | b) | If a displacement field is described as follows. u=( -x2+2y2+6xy)10-4 and v= ( 3x+6y-y2)10-4. How could you determine the strain components at the point x=1 and y=0. | 5M | BL3 | CO1 |
| 4 |  | An axial Load P = 300 KN is applied at 20°C to the rod as shown in Fig. The temperature is then raised 60°C. determine the nodal displacements and element stresses | 10M | BL3 | CO2 |
| OR Page 1 of 3 |  |  |  |  |  |
| 5 |  | Evaluate the element and Global Stiffness matrix for the beam element matrix for beam element as shown in the fig | 10M | BL3 | CO2 |
| 6 |  | Evaluate The Shape Functions N1, N2 and N3, At The Interior Point P for The Triangular Element Shown in Fig. also find the jacobian for triangular element | 10M | BL3 | CO3 |
| OR |  |  |  |  |  |
| 7 |  | A long bar of rectangular cross section, having thermal conductivity of 1.5 W/m °C is subjected to the boundary conditions shown in Fig. Two opposite sides are maintained at a uniform temperature of 180°C; one side is insulated, and the remaining side is subjected to a convection process with Tα = 25°C and h = 50 W/m2 0C. Determine the temperature distribution in the bar. | 10M | BL4 | CO3 |
| 8 |  | Derive the Element Stiffness matrix for the quadrilateral element | 10M | BL3 | CO4 |
| OR Page 2 of 3 |  |  |  |  |  |
| 9 | a) | In a quadrilateral element, the nodal coordinates in cm are given x1 = 6, x2= 2, x3 = 3, x4= 9, y1 = 8 y2= 5 y3 = 2 y4= 5, determine the global coordinates corresponding to ξ = -1.0 and η = 0.75 on the parent element | 5M | BL3 | CO4 |
|  | b) | For the isoparametric, quadrilateral element shown in figure determine the jocobian matrix at ξ = 1/4 and η = 1/4 | 5M | BL3 | CO4 |
| 10 | a) | Differentiate the static and Dynamic Analysis of structural problems | 5M | BL2 | CO5 |
|  | b) | Explain the evaluation of Eigen values and Eigen vectors using characteristic polynomial technique. | 5M | BL3 | CO5 |
| OR |  |  |  |  |  |
| 11 |  | Compute the natural frequencies for the stepped bar shown in figure. Take E = 2 x 105 N/mm2 and density = 1000 kg/m3. | 10M | BL4 | CO5 |

---oo0oo---

Page 3 of 3