| H.T NO. |  |  | R | 2 |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: B52107

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

I M.Tech II Semester Regular/Supplementary Examinations August/September-2025

ADVANCED HEAT AND MASS TRANSFER

(TE)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 1M = 10Marks

|  |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | What is Fourier’s law of conduction? | 1 | 1 | 1 |
|  | b) | What is thermal resistance? | 1 | 1 | 1 |
|  | c) | What is meant by transient heat conduction? | 1 | 2 | 1 |
|  | d) | Define forced convection | 1 | 2 | 1 |
|  | e) | Define Nusselt number and Prandtl number | 1 | 3 | 1 |
|  | f) | What is laminar flow? | 1 | 3 | 1 |
|  | g) | Define the term boiling | 1 | 4 | 1 |
|  | h) | List the any two assumptions of Nusselt theory of laminar flow film condensation on a vertical plate | 1 | 4 | 1 |
|  | i) | Define radiation heat transfer | 1 | 5 | 1 |
|  | j) | Define Stefan boltzman law. | 1 | 5 | 1 |

PART- B 			5 x 10M = 50Marks

|  |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
| 2 |  | Derive general Heat conduction equation for Cartesian coordinates. | 10 | 1 | 4 |
| OR |  |  |  |  |  |
| 3 | a) | Explain the analytical method for solving 2D steady state heat conduction problem. | 5 | 1 | 2 |
|  | b) | A plate 2cm thick and 10 cm wide is used to heat a fluid at 300C.The heat generation rate inside the plate is 7x 106 W/m3.Determine the heat transfer coefficient to maintain the temperature of the plate below 1800C.Given k (plate)=26 W/m0C. Neglect heat losses from the edge of the plate | 5 | 1 | 3 |
| 4 | a) | What do you understand by lumped system analysis? Infer an expression for temperature distribution in a body during Newtonian heating or cooling. | 5 | 2 | 4 |
|  | Page 1 of 2 |  |  |  |  |
|  | b) | Brass wire of 0.5 mm diameter is annealed in an annealing furnace at a temperature of 500 oC. Determine the time required for the wire to reach the temperature of 200oC if the temperature of the wire before passing into furnace is 30oC. h= 30 W/m2-K, ρ= 8600 kg/m2, Cp= 0.4 kJ/kg-K for brass Assume the length of the wire is 10 metres. | 5 | 2 | 3 |
| OR |  |  |  |  |  |
| 5 |  | Air flows over a thin plate with a velocity of 2.5 m/s. The plate is 100 cm x 100 cm. Estimate the boundary layer thickness at the training edge of the plate and the force necessary to hold the plate. Take the following air properties: µ=0.86 x 10-5 N s/m2, ρ=1.12 kg/m3 | 10 | 2 | 3 |
| 6 |  | Explain the development of hydrodynamic and thermal boundary layer for flow over a flat plate | 10 | 3 | 2 |
| OR |  |  |  |  |  |
| 7 | a) | Discuss about the constant heat flux boundary conditions. | 5 | 3 | 2 |
|  | b) | Air at 30oC flows with a velocity of 2.8 m/s over a plate 1000mm(length) x 600mm (width) x 25 mm (thickness). The top surface of the plate is maintained at 90oC.If the thermal conductivity of the plate material is 25 W/m oC, calculate the Heat lost by the plate. The thermo-physical properties of air at mean film temperature (90+30)/2=60OC are: ρ=1.06 kg/m3; Cp=1.005 kJ/kg K; k=0.02894 W/m oC ; ν = 18.97 x 10-6 m2/s; Pr=0.696. | 5 | 3 | 3 |
| 8 |  | Explain about the approximate analysis on laminar free convection heat transfer | 10 | 4 | 2 |
| OR |  |  |  |  |  |
| 9 |  | Illustrate the regimes of pool boiling with a neat sketch | 10 | 4 | 3 |
| 10 |  | Infer an expression for radiation heat exchange for two gray surfaces connected by single refractory surface | 10 | 5 | 4 |
| OR |  |  |  |  |  |
| 11 | a) | Discuss about the fick’s law of diffusion | 5 | 5 | 2 |
|  | b) | Air at 1 atm and 25oC, containing small quantities of iodine laws with a velocity of 6.2 m/s inside a 35 mm diameter tube. Calculate mass transfer coefficient for iodine. The thermophysical properties of air are:ν =15.5x 10-6 m2/s; D=0.82 X 10-5 m2/s | 5 | 5 | 3 |

---oo0oo---

Page 2 of 2