| H.T NO. |  |  | R | 2 |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Course Code: B52122

MLR INSTITUTE OF TECHNOLOGY

(An Autonomous Institute)

II M.Tech I Semester Regular/Supplementary Examinations August/September-2025

FINITE ELEMENT ANALYSIS

(TE)

Time: 3 Hours.								Max. Marks: 60

Note: 1. This question paper contains two parts A and B.

2. Part -A is Compulsory which carries 10 marks. Answer all Questions in part A.

3. Part -B consists 5units.Answer any one question from each unit. Each question 
              carries 10 marks and may have a, b, c as sub questions.

PART- A 			10 x 1M = 10Marks

|  |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | What are the properties of a stiffness matrix? | 1 | 1 | 1 |
|  | b) | Differentiate truss and beam. | 1 | 1 | 2 |
|  | c) | What are isoparametric elements? | 1 | 2 | 1 |
|  | d) | What are the assumptions in the Constant Strain Triangle (CST) element? | 1 | 2 | 1 |
|  | e) | What is the significance of eigenvalues in dynamic analysis? | 1 | 3 | 1 |
|  | f) | Explain the term "thermal force vector" in heat transfer problems. | 1 | 3 | 1 |
|  | g) | What are the limitations of Kirchhoff plate theory? | 1 | 4 | 1 |
|  | h) | What is geometric nonlinearity in finite element analysis? | 1 | 4 | 1 |
|  | i) | What is the advantage of BEM over FEM | 1 | 5 | 1 |
|  | j) | Explain the term "C(p)m" in electrostatic BEM problems | 1 | 5 | 1 |

PART- B 			5 x 10M = 50Marks

|  |  |  | M | CO | BL |
| --- | --- | --- | --- | --- | --- |
| 2 | a) | Derive the shape functions for a quadratic bar element | 5 | 1 | 3 |
|  | b) | In fig. a load P = 60 X 103 N is applied. Determine the displacement field, stress and support reactions in the body. Take E = 20 X 103 N/mm2. | 5 | 1 | 3 |
| OR |  |  |  |  |  |
| 3 | a) | Explain the Principle of Minimum Potential Energy and its application | 5 | 1 | 2 |
|  | b) | Derive the stiffness matrix for a beam element | 5 | 1 | 3 |
| Page 1 of 2 |  |  |  |  |  |
| 4 | a) | Derive the stiffness matrix for a Constant Strain Triangle (CST) element | 5 | 2 | 3 |
|  | b) | Evaluate the shape functions N1, N2, and N3 at the interior point P for the triangular element shown in Fig | 5 | 2 | 3 |
| OR |  |  |  |  |  |
| 5 | a) | Discuss the formulation of the gradient matrix and constitutive matrix in 2D elasticity problems | 5 | 2 | 2 |
|  | b) | Determine the shape functions for a quadrilateral isoparametric element | 5 | 2 | 3 |
| 6 |  | Calculate the temperature distribution in a 1D rectangular cross-section fin 8 cm long, 4 cm wide, and 1 cm thick. Assume that convective heat loss occurs from the end of the fin. Take K= 3W/cm-K, h=0.1 W/cm2-K and Tα= 20oC. Tip temperature is 100°C. | 10 | 3 | 3 |
| OR |  |  |  |  |  |
| 7 | a) | Discuss the applications of eigenvalues and eigenvectors in the dynamic analysis | 5 | 3 | 2 |
|  | b) | Differentiate lumped and consistent mass matrices in dynamic analysis | 5 | 3 | 3 |
| 8 |  | Differentiate Kirchhoff and Mindlin plate theories. What are the advantages and limitations? | 10 | 4 | 3 |
| OR |  |  |  |  |  |
| 9 | a) | How are boundary conditions applied in plate bending problems? | 5 | 4 | 3 |
|  | b) | Explain material nonlinearities in FEA of solids. | 5 | 4 | 2 |
| 10 | a) | Explain the advantages and limitations of BEM | 5 | 5 | 2 |
|  | b) | Explain the Boundary Element Approach in solving potential problems | 5 | 5 | 2 |
| OR |  |  |  |  |  |
| 11 |  | Describe the step-by-step numerical implementation of BEM for solving electrostatic problems | 10 | 5 | 3 |

---oo0oo---

Page 2 of 2